EXAMPLE 3-3B

Shear and Moment Diagrams of a Cantilever Beam Using Singularity Functions

Problem	Determine and plot the shear and moment functions for the cantilever beam with a concentrated load shown in Figure 3-22b.

Given	Beam length

l = 10;  % in

Distance to load w

a = 4;   % in

Applied force

F = 40;  % lbf

Assumptions	The weight of the beam is negligible compared to the applied load and so can be ignored.

Solution	See Figures 3-22b and 3-25, M-Book file Ex03-03.doc and M-File Ex03_03.m.

1	Write an equation for the load function in terms of equations 3.17 and integrate the resulting function twice using equations 3.18 to obtain the shear and moment functions.  Note use of the unit doublet function to represent the moment at the wall.  For the beam in Figure 3-22b,

q(x) = -M1<x - 0>-2 + R1<x - 0>-1 - F<x - a>-1	(a)

V(x) = -M1<x - 0>-1 + w<x - 0>0 - F<x - a>0  + C1	(b)

M(x) = -M1<x - 0>0 + R1<x - 0>1 - F<x - a>1 + C1 x + C2	(c)

The reaction moment M1 at the wall is in the z direction and the forces R1 and F are in the y direction in equation (b).  All moments in equation (c) are in the z direction.

2	Because the reactions have been included in the loading function, the shear and moment diagrams both close to zero at each end of the beam, making C1 = C2 = 0.

3	The reaction force R1 and the reaction moment M1 can be calculated from equations (b) and (c) respectively by substituting the boundary conditions x = l+, V = 0, M = 0.  Note that we can substitute l for  l+ since their difference is vanishingly small.

V(l) = -M1<l - 0>-1 + R1<l - 0>0 - F<l - a>0   = 0	(d)

V = R1 – F = 0

R1 = F  % lbf

R1 =

40

M(l) = -M1<l - 0>0 + R1<l - 0>1 - F<l - a>1  = 0	(e)

M = -M1 + R1 l – F (l-a) = 0

M1 = R1 * l - F * (l - a)   % lbf

M1 =

160

Since w, l, and a are known from the given data, equation (e) can be solved for R1, and this result substituted into equation (e) to find M1.  Note that equation (d) is just ΣFy = 0, and equation (e) is ΣMz = 0.

4	To generate the shear and moment functions over the length of the beam, equations (b) and (c) must be evaluated for a range of values of x from 0 to l, after substituting the above values of C1, C2, R1, and M1 in them.  For a Matlab solution, include a logical evaluation of the form (x >= z) to effect a step change.  This evaluates to a value of zero when x is less than the dummy variable z, and a value of one when it is greater than or equal to z.  It will have the same effect as the singularity function.

Range of x

x = 0:0.01*l:l;  % in

Write the shear and moment equations in Matlab form, including the logical evaluation (x>=z) as a multiplying factor to get the effect of the singularity functions.

V = R1.*(x>=0).*(x-0).^0 - F.*(x>=a).*(x-a).^0;    % lbf

M = -M1.*(x>=0).*(x-0).^0 + R1.*(x>=0).*(x-0).^1 - F.*(x>=a).*(x-a).^1;   % lbf in

subplot(1,2,1)

plot(x,V)

title('Shear Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Shear, V [lbf] (y-direction)')

ylim([-5 45])

subplot(1,2,2)

plot(x,M)

title('Moment Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Moment, M [lbf in] (z-direction)')

ylim([-200 50])

FIGURE 3-25. Example 3-3 Graphs

5	The largest absolute values of the shear and moment functions are of interest for the calculation of stresses in the beam.  The graphs show that the shear force and the moment are both largest at x = 0. The function values of these points can be calculated from equations (b) and (c) respectively by substituting x = 0 and evaluating the singularity functions:

Vmax = V(0) = R1<0 - 0>0 - F<0 - a>0 = R1

Vmax = R1   % lbf

Vmax =

40

Mmax = M(0) = -M1<0 - 0>0 + R1<0 - 0>1 - F<0 - a>1 = -M1

Mmax = abs(-M1) %lbf in

Mmax =

160