EXAMPLE 13-4A

Design of a Helical Compression Spring for Cyclic Loading:

An Alternate Approach

Problem	Design a compression spring for a dynamic load over a known deflection.

Given	Minimum force

Fmin = 60;       % lbf

Maximum force

Fmax = 150;      % lbf

Working deflection

DELTAy = 1.00;   % in

Forcing frequency

ff = 1000;       % cpm

Life (one shift)

L = 10;          % yr

Assumptions	Music wire (ASTM A228) will be used since the loads are dynamic.  Peening will be used to obtain a higher endurance strength.

Shear modulus

G = 11.5e6;     % psi

Solution	See M-Book Ex13-04A.doc and M-File Ex13_04A.m.

1	Find the the number of cycles that the spring will see over its design life.

Cycle life

Nlife = ff * 2080 * L * 60   % cycles

Nlife =

1.2480e+009

This large a number requires that an endurance limit for infinite life be used.

2	Find the the mean and alternating forces from equation 13.15a:

Alternating force

Fa = (Fmax - Fmin) / 2  % lbf

Fa =

45

Mean force

Fm = (Fmax + Fmin) / 2  % lbf

Fm =

105

3	We will derive a design equation for this problem that will yield a value for the wire diameter that is a function of only one parameter, the spring index C.  To start, we write the equation for the factor of safety against fatigue failure, equation 13.16b

Nfs=Ses⋅Sus−τiSes⋅τm−τi⋅τa	(c)

where Ses is given by equation 13.16c as

Ses=0.5⋅Sew⋅SusSus−0.5⋅Sew	(d)

and, from equations 13.5, 13.8b, and 13.9b,

τi=Ks⋅8⋅Fmin⋅Cπ⋅d2 τm=Ks⋅8⋅Fm⋅Cπ⋅d2 τa=Ks⋅8⋅Fa⋅Cπ⋅d2	(e)

From equations 13.3 and 13.4, the ultimate shear strength is

Sus=0.67⋅A⋅db	(f)

where A and b are taken from Table 13-4.

Substituting equations d, e and f into c and solving for the diameter d yields

d=8⋅C⋅Nfs0.67⋅π⋅A⋅Ks⋅Fm−Nfs−1Nfs⋅Ks⋅Fmin+0.670.5⋅A⋅dbSew−1⋅Kw⋅Fa12+b	(g)

Once we choose a material for the wire, the only unknown in this equation is the spring index C.

4	Assume a spring index of 7, then

Spring index

C = 7;

Direct shear factor

Ks = 1 + 0.5 / C

Ks =

1.0714

Wahl factor

Kw = (4*C - 1)/(4*C - 4) + 0.615 / C

Kw =

1.2129

5	From Tables 13-4 for A228 wire and from equation 13.12 we have

A = 184.649 * 1000; % psi

b = -0.1625;        % A and b from Table 13-4 for A228 music wire

Sew = 67.5 * 1000   % psi

Sew =

67500

6	Using these values and equation g we can solve for the required wire diameter by iteration.  In order to compare this solution with Example 13-4, let

Nfs = 1.3;

FUNC = @(d) d-(((8*C*Nfs)/(0.67*pi*A))*...

((Ks*Fm-(Nfs-1)/Nfs*Ks*Fmin)+...

((0.67/0.5)*((A*d^b)/Sew)-1)*Kw*Fa))^(1/(2+b));

d = fzero(FUNC,[0.19 0.22]) % solve for d over 0.19 < d < 0.22 in

d =

0.2091

This is not a preferred diameter as given in Table 13-2, so we will use the closest diameter.  Let

d = 0.207;   % in (closest preferred diameter)

7	Calculate the mean coil diameter D from equation 13.5

Mean coil diameter

D = C * d   % in

D =

1.4490

8	Calculate the shear stress τi at the initial deflection (lowest defined force), and the mean stress τm.

Stress at Fmin

TAUi = Ks * (8 * Fmin * D) / (pi * d^3)   % psi

TAUi =

2.6743e+004

Stress at Fm

TAUm = Ks * (8 * Fm * D) / (pi * d^3)   % psi

TAUm =

4.6800e+004

9	Calculate the alternating shear stress τa in the coil.

Alternating stress

TAUa = Kw * (8 * Fa * D) / (pi * d^3)   % psi

TAUa =

2.2705e+004

10	The spring rate is defined from the two specified forces at their relative deflection.

Spring rate

k = (Fmax - Fmin) / DELTAy   % lbf / in

k =

90

11	To get the defined spring rate, the number of active coils must satisfy equation 13.7, solving for Na yields:

Number of active coils

Na = (d^4 * G) / (8 * D^3 * k)

Na =

9.6392

Na = 9.75;  % rounding to nearest 1/4 coil

Note that we round it to the nearest 1/4 coil as the manufacturing tolerance cannot achieve better than that accuracy.  Having rounded the number of active coils, we must now calculate the spring rate using equation 13.7:

Corrected spring rate

k = (d^4 * G) / (8 * D^3 * Na)  % lbf / in

k =

88.9773

12	Assume squared and ground ends making the total number of coils, from Figure 13-9:

Total coils

Nt = Na + 2

Nt =

11.7500

13	The shut height can now be determined.

Ls = d*Nt   % in

Ls =

2.4322

14	The initial deflection to reach the smaller of the two loads is

yinit = Fmin / k    % in

yinit =

0.6743

15	Assume a clash allowance of 15% of the working deflection:

DELTAyClash = 0.15 * DELTAy  % in

DELTAyClash =

0.1500

16	The free length (see Figure 13-8) can now be found from

Lf = Ls + DELTAyClash + DELTAy + yinit   % in

Lf =

4.2566

17	The deflection to the shut height is

yshut = Lf - Ls     % in

yshut =

1.8243

18	The force at this shut height is

Fshut = k * yshut   % lbf

Fshut =

162.3240

19	The shut-height stress and safety factor are

Stress at Fshut

TAUshut = Ks * (8 * Fshut * D) / (pi * d^3)   % psi

TAUshut =

7.2351e+004

Shear yield strength

Sys = 0.60 * A * d^b    % psi (assumes d [=] in

Sys =

1.4310e+005

Safety factor at shut height

Ns = Sys / TAUshut

Ns =

1.9779

which is acceptable.

20	To check for buckling, two ratios need to be calculated, Lf / D and ymax / Lf.

Slenderness ratio

sr = Lf / D

sr =

2.9376

Deflection ratio

yprm = (yinit + DELTAy) / Lf

yprm =

0.3934

Take these two values to Figure 13-14 and find that their coordinates are safely within the zones that are stable against buckling for either end-condition case.

21	The inside and outside coil diameters are

Inside coil diameter

Di = D - d  % in

Di =

1.2420

Outside coil diameter

Do = D + d  % in

Do =

1.6560

22	The weight of the spring's active coils is found from equation 13.11b and is

Weight density

RHO = 0.285;  % lbf / in3

Weight

Wa = (pi^2 * d^2 * D * Na * RHO) / 4    % lbf

Wa =

0.4257

23	The natural frequency of this spring is found from equation 13.11a and is:

g = 32.174 * 12     % in / sec2

fn = 0.5 * sqrt((k*g)/Wa)       % Hz

fn = 0.5 * sqrt((k*g)/Wa) * 60  % cpm

g =

386.0880

fn =

142.0377

fn =

8.5223e+003

The ratio between the natural frequency and the forcing frequency is

FRatio = fn / ff

FRatio =

8.5223

which is sufficiently high.

24	We now have a complete design specification for this A228-wire spring:

Wire diameter

d   % in

d =

0.2070

Outside diameter

Do  % in

Do =

1.6560

Total coils (ends squared and ground)

Nt  % coils

Nt =

11.7500

Free length

Lf  % in

Lf =

4.2566