EXAMPLE 7-2

Stresses in Cylindrical Contact

Problem	An overhead crane wheel runs slowly on a steel rail.  What is the size of the contact patch between wheel and rail and what are the stresses?  What is the depth of the maximum shear stress?

Given	Wheel diameter

d = 12.0;    % in

R1 = d/2     % in

R1 =

6

Rail curvature

R2 = inf;    % in

Width of wheel

L = 0.875;   % in

Radial load

F = 5000;    % lbf

The parts are hardened steel.  Therefore

E = 30e6;    % psi

v = 0.28;

Assumptions	The rotational speed is sufficiently slow that this can be considered a static loading problem.

Solution	See M-Book Ex07-02.doc and M-File Ex07_02.m.

1	We need to first determine the size of the contact patch, for which we need to find the geometry constant and material constants from equations 7.15a and 7.9a.

Geometry constant

B = (1/2) * ((1/R1) + (1/R2))   % in^-1

B =

0.0833

Note the infinite radius for R2.

Material constants

m1 = (1-v^2)/E  % psi^-1

m2 = m1         % psi^-1

m1 =

3.0720e-008

m2 =

3.0720e-008

Note both materials are the same in this example.  The material and geometry constants can now be used in equation 7.15b.

Contact patch radius

a = ((2/pi)*(((m1+m2)/B)*(F/L)))^(1/2)    % in

a =

0.0518

where a is the half-width (radius) of the contact patch.  The rectangular contact patch area is

Contact area

A = 2 * L * a    % in2

A =

0.0906

2	The average and maximum contact pressure can now be found from equations 7.14b and c.

Average pressure

Pavg = F/A/1000              % ksi

Pavg =

55.1689

Maximum pressure

Pmax = (2*F)/(pi*a*L)/1000   % ksi

Pmax =

70.2433

3	The maximum normal stresses in the center of the contact patch at the surface are then found using equations 7.11a.

Axial

SIGzmax = -Pmax         % ksi

SIGzmax =

-70.2433

In-plane

SIGymax = -2*v*Pmax     % ksi

SIGymax =

-39.3362

4	The maximum shear stress and its location (depth) are found from equations 7.17b.

Maximum shear stress

TAU13max = 0.304*Pmax   % ksi

TAU13max =

21.3540

Depth at maximum shear stress

z_taumax = 0.786*a      % in

z_taumax =

0.0407

5	All of the stresses found exist on the z-axis and the normal stresses are principle.  These stresses apply to the wheel and rail as both are steel.