EXAMPLE 14-3

Preloaded Fasteners in Dynamic Loading

Problem	Repeat Example 14-2 with a fluctuating load applied to the joint.  Determine a suitable bolt size and preload for the joint shown in Figure 14-23.  Find its safety factors against fatigue, yielding, and separation.  Determine the optimum preload as a percentage of proof strength to maximize the fatigue, yielding, and separation safety factors.

Given	The joint dimensions are:

Cylinder diameter

D = 1.00;    % in

Clamp length

lm = 2.00;   % in

Applied load

Pmin = 0;    % lbf

Pmax = 2000; % lbf

Assumptions	The bolt has rolled threads.  Both of the clamped parts are steel.  The effects of the flanges on the joint stiffness will be ignored.  A preload of 90% of the bolts proof strength will be applied as a first trial.  Use 99% reliability and an operating temperature of 300 deg F.

Young's modulus

E = 30e6;    % psi

Preload fraction of proof strength

fp = 0.9;

Solution	See Figures 14-27 to 14-29, M-Book Ex14-03.doc and M-File Ex14_03.m.

1	Again, there are too many unknown variables to solve the necessary equations in one pass.  Trial values must be chosen for various parameters and iteration used to find a good solution.  We actually made several iterations to solve this problem, but will only present one in the interest of brevity.  The trial values used here have already been massaged to reasonable values.

2	The bolt diameter is the principal trial value to be chosen along with a thread series and a bolt class to define the proof strength.  We choose a 5/16-18 UNC-2A steel bolt of SAE class 5.2 based on its successful use in the static-loading problem of Example 14-2 on which this example is based.  For a clamp length of 2 in, assume a bolt length of 2.5 in to allow sufficient protrusion for the nut.  The preload is taken at 90% of proof strength as assumed above.

Bolt diameter

d = 0.3125;  % in

Bolt length

l = 2.500;   % in

Shank area

Ab = (pi * d^2) / 4   % in2

Ab =

0.0767

3	The proof strength, tensile-stress area, preload force, bolt stiffness, material stiff- ness, and joint constant are all the same as were found in Example 14-2 for the 90% preload factor.  See that example for details.  In summary they are

Sp = 85 * 1000   % psi

At = 0.05243;    % in2

Fi = 4010.95;    % lbf

kb = 1.059e6;    % lbf / in

km = 1.063e7;    % lbf / in

C = 0.09056;

Sp =

85000

4	The portions of the peak fluctuating load P felt by the bolt and the material and the resulting loads in bolt and material after the load is applied are also the same as in the previous example:

Pb = 181.12;     % lbf

Pm = 1818.87;    % lbf

Fb = 4192.08;    % lbf

Fm = 2192.08;    % lbf

5	Because these loads are fluctuating, we need to calculate the mean and alternating components of the force felt in the bolt.  Figure 14-27 shows the load-deflection diagram for this problem, drawn to scale with the above forces applied.  The shallow sine wave between the initial force line A and the maximum bolt force line B is the only fluctuating load felt by the bolt.  The mean and alternating forces are then

Falt = (Fb - Fi) / 2    % lbf

Fmean = (Fb + Fi) / 2   % lbf

Falt =

90.5650

Fmean =

4.1015e+003

Note how little of the 0-2000 lb fluctuating force is felt by the bolt.

6	The nominal mean, alternating, maximum, minimum stresses in the bolt are:

SIGanom = Falt / At     % psi

SIGmnom = Fmean / At    % psi

SIGanom =

1.7274e+003

SIGmnom =

7.8228e+004

SIGmaxnom = SIGanom + SIGmnom   % psi

SIGminnom = SIGmnom - SIGanom   % psi

SIGmaxnom =

7.9956e+004

SIGminnom =

7.6501e+004

7	The fatigue stress-concentration factor Kf = 3.0 for rolled threads is taken from Table 14-8.  The mean stress-concentration factor Kfm is found using a functon M-file called KfmFUNC.m. A complete listing of this M-file is provided at the end of this Example.

Kf = 3.0;

Sy = 92 * 1000  % psi

Kfm = KfmFUNC(Kf,Sy,SIGmaxnom,SIGminnom,SIGanom,SIGmnom)

Sy =

92000

Kfm =

1.1098

8	The local mean and alternating stresses in the bolt are then:

SIGalt = Kf * Falt / At     % psi

SIGmean = Kfm * Fmean / At  % psi

SIGalt =

5.1821e+003

SIGmean =

8.6818e+004

9	The stress at the initial preload is

SIGinit = Kfm * Fi / At     % psi

SIGinit =

8.4901e+004

10	An endurance strength must be found for this material.  Using the methods of Section 6.6 we find for

Sut = 120 * 1000    % psi

Sprme = 0.5 * Sut   % psi

Sut =

120000

Sprme =

60000

From the tables and formulas in Section 6.6 we have:

Cload = 0.70;

Csize = 1;

Csurf = 0.76;

Ctemp = 1;

Creliab = 0.81;

and the endurance limit is

Se = Cload * Csize * Csurf * Ctemp * Creliab * Sprme    % psi

Se =

2.5855e+004

11	The corrected endurance strength and the ultimate tensile strength are used in equation 14.16 to find the safety factor from the Goodman line.

Nf = Se*(Sut - SIGinit) /...

(Se*(SIGmean - SIGinit) + Sut*SIGalt)

Nf =

1.3516

The modified-Goodman diagram for this stress state is shown in Figure 14-28.

12	The maximum static stress and the safety factor against yielding are the same as in the previous example where:

Sy = 92 * 1000          % psi

SIGb = 79.04 * 1000     % psi

Sy =

92000

SIGb =

79040

and

Ny = Sy / SIGb

Ny =

1.1640

13	The load required to separate the joint and the safety factor against joint separation are found from equations 14.14c and 14.14d.

Nsep = Fi / (Pmax * (1 - C))

Nsep =

2.2052

14	The fatigue and separation safety factors are acceptable.  The yielding safety factor is low.  We will solve the model for the entire range of possible preloads from zero to 100 percent of proof strength and then these three safety factors can be plotted versus percent preload.  The results are shown in Figure 14-29.

15	The preload required is found from

Finit = 0.90 * Sp * At  % lbf

Finit =

4.0109e+003

16	The recommended design is then a 5/16-18 UNC-2A, grade 5.2 bolt, 2.5 in long, preloaded to 90% of proof strength with a force of  4011 lb.  The safety factors against overload is 2.2, against yielding is 1.2, and against fatigue failure is 1.4.

Function M-File: KfmFUNC.m

function ret = KfmFUNC(K,S,SIGmax,SIGmin,SIGanom,SIGmnom)

if K * abs(SIGmax) < S

ret = K;

end

if K * abs(SIGmax) > S

ret = (S - K*SIGanom) / abs(SIGmnom);

end

if K * abs(SIGmax - SIGmin) > (2 * S);

ret = 0

end