EXAMPLE 15-1

Design of a Disk Clutch

Problem	Determine a suitable size and required force for an axial disk clutch.

Given	Transmitted power

P = 7.5;            % hp

P = 7.5 * 6600      % in lbf / sec

P =

49500

Clutch speed

OMEGA = 1725;       % rpm

OMEGA = 1725 * 2 * pi / 60   % rad / sec

OMEGA =

180.6416

Service factor

Nsf = 2;

Assumptions	Using a uniform-wear model, N = 1.  Assume a single-dry-disk with a molded lining.

N = 1;

Solution	See M-Book Ex15-01.doc and M-File Ex15_01.m.

1	The service factor of 2 requires derating the clutch by that factor so we will design for Nsf . P = 15 hp instead of 7.5.  Find the torque required for that power at the design rpm.

T = (Nsf * P) / OMEGA   % lbf in

T =

548.0466

2	Find the coefficient of friction and maximum recommended pressure for a dry, molded material from Table 15-1.  Use the average of the ranges of values shown:

pmax = 225; % psi

MU = 0.35;

3	Substitute equation 15.7 relating ri to ro for maximum torque into equation 15.5b to get

T=π⋅μ⋅ri⋅pmax⋅ro2−ri2=π⋅μ⋅13⋅ro⋅pmax⋅ro2−13⋅ro2

Solving for ro

ro = (T/((2/3) * (sqrt(1/3) * pi * MU * pmax)))^(1/3)   % in

ro =

1.7921

4	From equation 15.7:

ri = sqrt(1/3) * ro     % in

ri =

1.0347

5	Using standard values (even fractions of an inch converted to decimal form), let the inside and outside radii be

ri = 1.000  % in (standard size)

ri =

1

ro = 29/16  % in (standard size)

ro =

1.8125

Then the axial force needed (from equation 15.5a) is:

F = 2 * pi * ri * pmax * (ro - ri)  % lbf

F =

1.1486e+003

6	The clutch specification is then a single disk with 3.625-in outside diameter and 2.000-in inside diameter, a molded lining with µdry >= 0.35, and an actuating force >= 1149 lb.