EXAMPLE 4-2

Determining Plane Stresses Using Mohr's Circles

Problem	A biaxial stress element as shown in Figure 4-2 has the given stress components.  Use Mohr's circles to determine the principal stresses.

Given	Normal stresses

SIGx = 40;   % ksi    (ksi = 1000 x psi)

SIGy = 20;   % ksi

Shear stress (ccw)

TAUxy = 10;  % ksi ccw

Solution	See Figures 4-2 and 4-6, M-Book Ex04-02.doc and M-file Ex04_02.m

1	Construct the Mohr-plane axes as shown in Figure 4-6 and label them σ and τ.

2	Lay off the given applied stress σx (as line OA) to scale along the normal stress (horizontal) axis.  Note that σx is a tensile (positive) stress in this example.

3	Lay off the given applied stress σy (as line OB) to scale along the normal stress axis.  Note that σy is also a tensile (positive) stress in this example and so lies in the same direction as σx along the σ axis.

4	Figure 4-2 shows that the shear stresses τxy create a ccw couple on the element.  This couple is baklanced for equilibrium by the cw couple provided by the shear stresses τyx.  Recall that both these shear stresses, τxy and τyx , are equal according to equation 4.2 and are positive according to the stress sign convention.  But, instead of using the stress sign convention, they are plotted on the Mohr circle according to the rotation that they imply to the element, using Mohr's left-handed sign convention of cw+ and ccw-.

5	Draw a vertical line downward (ccw-) from the tip of σx (as line AC) to represent the scaled magnitude of  τxy.  Draw a vertical line upward (cw+) from the tip of σy (as line BD) to represent the scaled magnitude of τyx.

6	The diameter of one Mohr's circle is the distance from point C to point D.  Line AB bisects CD.  Draw the circle using this intersection as the center and the distance from the intersection to C or D as the radius.

Center at

SIGcent = (SIGx + SIGy) / 2 % ksi

SIGcent =

30

Radius

R = sqrt(((SIGx - SIGy) / 2)^2 + TAUxy^2)   % ksi

R =

14.1421

7	Two of the three principal normal stresses are then found at the two intersections that this Mohr's circle makes with the normal stress axis at points P1 and P2:

SIG1 = SIGcent + R  % ksi

SIG2 = SIGcent - R  % ksi

SIG1 =

44.1421

SIG2 =

15.8579

Note that if we stop at this point, the maximum shear stress appears to be

TAU12 = R   % ksi

TAU12 =

14.1421

as defined by the projection of a horizontal tangent from the top of one circle to the τ axis, as shown in Figure 4-6b.

8	Since there were no applied stresses in the z direction in this example, it is a 2-D stress state and the third principal stress, σ3, is known to be zero, thus is located at point O, also labeled P3.

SIG3 = 0    % ksi

SIG3 =

0

9	There are still two other Mohr's circles to be drawn.  The three Mohr's circles are defined by the diameters (σ1 - σ3), (σ1 - σ2), and (σ2 - σ3), which in this case, are the lines P1P3, P1P2, and P2P3 as shown in Figure 4-6.

10	Extend horizontal tangent lines from the top and bottom extremes of each Mohr's circle to intersect the shear (vertical) axis.  This determines the value of the principal shear stress associated with each pair of principal normal stresses:

TAU13 = (SIG1 - SIG3) / 2  % ksi

TAU12 = (SIG1 - SIG2) / 2  % ksi

TAU23 = (SIG2 - SIG3) / 2  % ksi

TAU13 =

22.0711

TAU12 =

14.1421

TAU23 =

7.9289

The largest of these is

TAUmax = TAU13   % ksi

TAUmax =

22.0711

not the value

TAU12           % ksi

TAU12 =

14.1421

found in step 7.

11	Note that it is always the circle lying between the largest and smallest principal stresses that determines the maximum shear stress.  In the previous example the zero principal stress was not the smallest of the three because one principal stress was negative.  In the present example, the zero principal stress is the smallest.  Thus, failing to draw all three circles would have lead to a serious error in the value of

TAUmax          % ksi

TAUmax =

22.0711