EXAMPLE 9-6

Determining the Energy Variation in a Torque-Time Function

Problem	Find the energy variation per cycle in a torque-time function that needs to be absorbed by a flywheel for smooth operation.

Given	A torque-time function that varies over its cycle as shown in Figure 9-22.  The torque varies during the 360 deg cycle about its average value.

Assumptions	The one cycle of torque variation shown is representative of the steady-state condition.  Energy delivered from the source to the load will be considered positive and energy returned from load to source, negative.

Solution

1	Calculate the average value of the torque-time function over one cycle using numerical integration. In this case it is 7020 lb-in.  (Note that in some cases the average value may be zero.)

2	Note that the integration on the left-hand side of equation 9.18c is done with respect to the average line of the torque function, not with respect to the θ axis.  (From the definition of the average, the sum of positive area above an average line is equal to the sum of negative area below that line.)  The integration limits in equation 9.18 are from the shaft angle θ, at which the shaft ω is a minimum to the shaft angle θ, at which ω is a maximum.

3	The minimum ω will occur after the maximum positive energy has been delivered from the motor to the load, i.e., at a point θ where the summation of positive energy (area) in the torque pulses is at its largest positive value.

4	The maximum ω will occur after the maximum negative energy has been returned to the load, i.e., at a point θ where the summation of energy (area) in the torque pulses is at its largest negative value.

5	To find these locations in θ corresponding to the maximum and minimum ω's and thus find the amount of energy needed to be stored in the flywheel, we need to numerically integrate each pulse of this function from crossover to crossover with the average line.  The crossover points have been labeled A, B, C, and D and the areas between them are shown in Figure 9-22.

6	The remaining task is to accumulate these pulse areas beginning at an arbitrary crossover (in this case point A) and proceeding pulse by pulse across the cycle.  Table 9-5 shows this process and the result.

7	Note in Table 9-5 that the minimum shaft speed occurs after the largest accumulated positive energy pulse (+20073 in-lb) has been delivered from the driveshaft to the system.  This delivery of energy slows the motor down.  The maximum shaft speed occurs after the largest accumulated negative energy pulse (6032 in-lb) has been received back from the load by the shaft.  This return of stored energy will tend to speed up the motor.  The total energy variation is the algebraic difference between these two extreme values, which in this example is 26105 in-lb.  This energy coming back from the load needs to be absorbed by the flywheel and later returned to the system within each cycle to smooth the variations in shaft speed.