EXAMPLE 3-4

Shear and Moment Diagrams of an Overhung Beam Using Singularity Functions

Problem	Determine and plot the shear and moment functions for the overhung beam with an applied moment and ramp load shown in Figure 3-22c.

Given	Beam length

l = 10;  % in

Distance to load w

a = 4;   % in

Applied moment

M1 = 20; % lbf in

Ramp slope

w = 10;  % lbf / in in

Assumptions	The weight of the beam is negligible compared to the applied load and so can be ignored.

Solution	See Figures 3-22c and 3-26, M-Book Ex03-04.doc and M-file Ex03_04.m.

1	Write an equation for the load function in terms of equations 3.17 and integrate the resulting function twice using equations 3.18 to obtain the shear and moment functions.  For the beam in Figure 3-22c,

q(x) = M1<x - 0>-2 + R1<x - a>-1 - w<x - a>1 + R2<x - l>-1	(a)

V(x) = M1<x - 0>-1 + R1<x - a>0 – (w/2)<x - a>2 + R2<x - l>0  + C1	(b)

M(x) = M1<x - 0>0 + R1<x - a>1 – (w/6)<x - a>3 + R2<x - l>1  + C1x + C2	(c)

2	As demonstrated in the previous two examples, the constants of integration C1 and C2 will always be zero if the reaction forces are included in the load function.  So we will set them to zero.

3	The reaction forces R1 and R2 can be calculated from equations (c) and (b) respectively by substituting the boundary conditions x = l+, V = 0, M = 0.  Note that we can substitute l for  l+ since their difference is vanishingly small.

M(l) = M1<l>0 + R1<l - a>1 – (w/6)<l - a>3 + R2<l - l>1 = 0	(d)

M = M1 + R1*(l-a) – (w/6)*(l-a)3 = 0

R1 = ((w * (l - a)^2) / 6) - (M1 / (l - a)) % lbf

R1 =

56.6667

V(l) = M1<l>-1 + R1<l - a>0 – (w/2)<l - a>2 + R2<l - l>0  = 0	(e)

V = R1 – (w/2)*(l-a)2 + R2 = 0

R2 = (w/2) * (l - a)^2 - R1 %lbf

R2 =

123.3333

Note that equation (d) is just ΣMz = 0, and equation (e) is ΣFy = 0

4	To generate the shear and moment functions over the length of the beam, equations (b) and (c) must be evaluated for a range of values of x from 0 to l, after substituting the above values of C1, C2, R1, and R2 in them.  For a Matlab solution, include a logical evaluation of the form (x>=z).  This will evaluate to a value of zero when x is less than the dummy variable z, and a value of one when it is greater than or equal to z.  It will have the same effect as the singularity function.

Range of x

x = 0:0.005*l:l;    % in

Write the shear and moment equations in Mathcad form, using the logical evaluation (x>=z) as a multiplying factor to get the effect of the singularity functions.

V = R1.*(x>=a).*(x-a).^0 - (w/2).*(x>=a).*(x-a).^2 +...

R2.*(x>=l).*(x-l).^0;    % lbf

M = M1.*(x>=0).*(x-0).^0 + R1.*(x>=a).*(x-a).^1 -...

(w/6).*(x>=a).*(x-a).^3 + R2.*(x>=l).*(x-l).^1; % lbf in

subplot(1,2,1)

plot(x,V)

title('Shear Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Shear, V [lbf] (y-direction)')

ylim([-150 100])

subplot(1,2,2)

plot(x,M)

title('Moment Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Moment, M [lbf in] (z-direction)')

ylim([0 200])

FIGURE 3-26. Example 3-4 Graphs

5	The largest absolute values of the shear and moment functions are of interest for the calculation of stresses in the beam.  The graphs (below) show that the shear force is largest at x = l and the moment has a maximum to the right of the beam center.  The value of x at Mmax can be found by setting V to 0 in equation (b) and solving for x.  (The shear function is the derivative of the moment function and so must be zero at each of the moment function's minima and maxima.)  Let the distance along the beam from the origin to the point where V = 0 be c,

V(c) = R1<c - a>0 – (w/2)<c - a>2 + R2<c - l>0 = 0	(f)

Solving for c

R1 = (w/2)*(c-a)2 = 0

c = sqrt((2*R1)/w) + a  % in

c =

7.3665

The maximum moment is found by evaluating equation (c) at x = c:

M(c) = M1<c>0 + R1<c - a>1 – (w/6)<c - a>3 + R2<c - l>1 = Mmax	(g)

Mmax = M1 + R1*(c-a) - (w/6)*(c-a)^3    % lbf in

Mmax =

147.1790