EXAMPLE 7-1

Stresses in a Ball Thrust Bearing

Problem	A ball thrust bearing with 7 balls is loaded axially across its races through the balls.  What is the size of the contact patch on a race and what are the stresses developed in balls and races?  What is the depth of the maximum shear stress in a ball?

Given	Ball diameter

d = 0.3937;      % in

R1 = d/2         % in

R1 =

0.1969

Number of balls

N = 7;

Race curvature

R2 = inf;        % in

Total axial load

Ftot = 151;      % lbf

Load per ball

F = Ftot / N     % lbf

F =

21.5714

The parts are hardened steel.  Therefore

E = 30e6;        % psi

v = 0.28;

Assumptions	The 7 balls share the load equally.  The rotational speed is sufficiently slow that this can be considered a static loading problem.

Solution	See M-Book Ex07-01.doc and M-File Ex07_01.m.

1	We need to first determine the size of the contact patch, for which we need to find the geometry constant and material constants from equations 7.9a and b.

Geometry constant

B = (1/2) * ((1/R1) + (1/R2))   % in^-1

B =

2.5400

Note the infinite radius for R2.

Material constants

m1 = (1-v^2)/E  % psi^-1

m2 = m1         % psi^-1

m1 =

3.0720e-008

m2 =

3.0720e-008

Note both materials are the same in this example.  The material and geometry constants can now be used in equation 7.9d.

Contact patch radius

a = ((3/8)*(((m1+m2)/B)*F))^(1/3)    % in

a =

0.0058

where a is the half-width (radius) of the contact patch.  The circular contact patch area is then

Contact area

A = pi * a^2    % in2

A =

1.0588e-004

2	The average and maximum contact pressure can now be found from equations 7.8c and d.

Average pressure

Pavg = F/A/1000     % ksi

Pavg =

203.7251

Maximum pressure

Pmax = (3/2)*Pavg   % ksi

Pmax =

305.5876

3	The maximum normal stresses in the center of the contact patch at the surface are then found using equations 7.11c and d.

Axial

SIGzmax = -Pmax                 % ksi

SIGzmax =

-305.5876

In-plane

SIGxmax = -((1+2*v)/2)*Pmax     % ksi

SIGymax = SIGxmax               % ksi

SIGxmax =

-238.3583

SIGymax =

-238.3583

4	The maximum shear stress and its location under the surface are found from equations 7.12b and c.

Maximum shear stress

TAUyzmax = (Pmax/2)*(((1-2*v)/2) +...

(2/9)*(1+v)*sqrt(2*(1+v)))   % ksi

TAUyzmax =

103.1528

Depth at maximum shear stress

z_taumax = a*sqrt((2+2*v)/(7-2*v))  % in

z_taumax =

0.0037

5	All of the stresses found so far exist on the centerline of the patch.  At the edge of the patch, at the surface, there will be a shear stress of

TAUxy = ((1-2*v)/3)*Pmax     % ksi

TAUxy =

44.8195

and a tensile stress of the same magnitude.

6	Since both parts are the same material, all of the stresses apply to both.