EXAMPLE 13-6

Design of a Helical Torsion Spring for Cyclic Loading

Problem	Design a torsion spring for a dynamic load over a given deflection.

Given	Minimum moment

Mmin = 50;          % lbf in

Maximum moment

Mmax = 80;          % lbf in

Working deflection

DELTATHETA = 0.25;  % rev

Life

L = inf;

Assumptions	Use unpeened music wire (ASTM A228).  Use 2-in-long straight, tangent ends.  The coil is loaded to close it.

Modulus of elasticity

E = 30e6;   % psi

Length of ends

L1 = 2;     % in

L2 = 2;     % in

Solution	See Figures 13-27, M-Book Ex13-06.doc and M-File Ex13_06.m.

1	Assume a trial wire diameter from the available sizes in Table 13-2.  Assume a spring index of 9 and calculate the mean coil diameter D from equation 13.5.

Wire diameter

d = 0.192;  % in

Spring index

C = 9;      % coils

Mean coil diameter

D = C * d   % in

D =

1.7280

2	Find the the mean and alternating moments:

Alternating moment

Ma = (Mmax - Mmin) / 2  % lbf

Ma =

15

Mean moment

Mm = (Mmax + Mmin) / 2  % lbf

Mm =

65

3	Find the Wahl bending factor for the inside surface Kbi and use it to calculate the maximum compressive stress in the coil at the inner surface.

Kbi = (4 * C^2 - C - 1) / (4 * C * (C - 1))

SIGimax = Kbi * (32*Mmax) / (pi*d^3)

Kbi =

1.0903

SIGimax =

1.2552e+005

4	Find the Wahl bending factor Kbo for the outside surface and calculate the maximum, minimum, alternating, and mean tensile stresses in the coil at the outer surface.

Kbo = (4 * C^2 + C - 1) / (4 * C * (C + 1))

SIGomin = Kbo * (32*Mmin)/(pi*d^3)        % psi

SIGomax = Kbo * (32*Mmax)/(pi*d^3)        % psi

SIGm = (SIGomax + SIGomin) / 2            % psi

SIGa = (SIGomax - SIGomin) / 2            % psi

Kbo =

0.9222

SIGomin =

6.6359e+004

SIGomax =

1.0617e+005

SIGm =

8.6267e+004

SIGa =

1.9908e+004

5	Find the ultimate tensile strength of this music-wire material from equation 13.3 and Table 13-4 and use it to find the bending yield strength from Table 13-13, assuming no stress relieving.

From Table 13-4, for A228 music wire

A = 184.649 * 1000; % psi

b = -0.1625;    % A and b from Table 13-4 for A228 music wire

Ultimate tensile strength

Sut = A * d^b       % psi (assumes d [=] in)

Sut =

2.4144e+005

Bending yield strength

Sy = 1.0 * Sut      % psi

Sy =

2.4144e+005

6	Find the wire bending endurance limit for unpeened springs from equation 13.33 and convert it to fully reversed endurance strength with equation 13.34b.

Wire endurance limit

Sewb = 45.0 / 0.577 *1000  % psi

Sewb =

7.7990e+004

Fully reversed endurance limit

Se = 0.5 * (Sewb*Sut) / (Sut - 0.5*Sewb)    % psi

Se =

4.6506e+004

7	The fatigue safety factor for the coils in bending is calculated from equation 13.34a.

Fatigue factor of safety

Nfb = (Se*(Sut-SIGomin))/(Se*(SIGm-SIGomin)+Sut*SIGa)

Nfb =

1.4204

8	The static safety factor against yielding is

Static factor of safety

Nyb = Sy / SIGimax

Nyb =

1.9235

These are both acceptable safety factors.

9	The spring rate is defined from the two specified moments at their relative deflection.

Spring rate

k = (Mmax - Mmin) / DELTATHETA   % lbf in / rev

k =

120

10	To get the defined spring rate, the number of active coils must satisfy equation 13.28, solving for Na yields:

Number of active coils

Na = (d^4 * E) / (10.8 * D * k)

Na =

18.2044

The ends contribute to the active coils from equation 13.26a as

Ne = (L1 + L2) / (3*pi*D)

Ne =

0.2456

and, from equation 13.26b, the number of body coils in the spring are

Nb = Na - Ne

Nb =

17.9588

11	The angular deflections at the specified loads from equation 13.27c are

THETAmin = 10.8 * (Mmin*D*Na) / (d^4 * E)            % rev

THETAminDeg = 10.8 * (Mmin*D*Na) / (d^4 * E) * 360   % deg

THETAmin =

0.4167

THETAminDeg =

150