EXAMPLE 5-1

Failure of Ductile Materials Under Static Loading

Problem	Determine the safety factors for the bracket rod shown in Figure 5-7 based on both the distortion-energy theory and the maximum shear theory and compare them.

Given	Yield strength

Sy = 47000;      % psi

Rod length

L = 6;           % in

Arm length

a = 8;           % in

Rod diameter

d = 1.5;         % in

Load

F = 1000;        % lbf

Assumptions	The load is static and the assembly is at room temperature.  Consider shear due to transverse loading as well as other stresses.

Solution	See Figures 5-7 and 4-33, M-Book Ex05-1.doc and M-File Ex05_01.m.  Also see Example 4-9 for a more complete explanation of the stress analysis for this problem.

1	The rod is loaded in both bending (as a cantilever beam) and in torsion. The largest tensile bending stress will be in the top outer fiber at point A.  The largest torsional shear stress will be all around the outer circumference of the rod.  (See Example 4-9 for more detail.)  First take a differential element at point A where both of these stresses combine as shown in Figure 4-33b.  Find the normal bending stress and torsional shear stress on point A using equations 4.11b and 4.24b, respectively.

Bending moment at point A

M = F*L                 % lbf in

M =

6000

Distance to outer fiber

c = 0.5 * d             % in

c =

0.7500

Moment of inertia

I = (pi * d^4) / 64     % in4

I =

0.2485

Torque carried by rod

T = F*a                 % lbf in

T =

8000

Polar moment of inertia

J = 2*I                 % in4

J =

0.4970

Normal bending stress

SIGx = (M*c)/I          % psi

SIGx =

1.8108e+004

Torsional shear stress

TAUxz = (T*c)/J         % psi

TAUxz =

1.2072e+004

2	Find the maximum shear stress and principal stresses that result from this combination of applied stresses at point A using equations 4.6.

Normal stress in z-direction

SIGz = 0                % psi

SIGz =

0

Maximum shear stress

TAUmax = sqrt(((SIGx - SIGz)/2)^2 + TAUxz^2)      % psi

TAUmax =

1.5090e+004

Principal stresses

SIG1 = (SIGx + SIGz) / 2 + TAUmax     % psi

SIG2 = 0    % psi

SIG3 = (SIGx + SIGz) / 2 - TAUmax     % psi

SIG1 =

2.4144e+004

SIG2 =

0

SIG3 =

-6.0361e+003

3	Find the von Mises effective stress from the principal stresses using equation 5.7a with σ2 = 0.

SIGprime = sqrt(SIG1^2 - SIG1*SIG3 + SIG3^2)    % psi

SIGprime =

2.7661e+004

4	The safety factor using the distortion-energy theory can now be found using equation 5.8a.

N = Sy / SIGprime           % no units

N =

1.6992

5	The safety factor using the maximum shear-stress theory can now be found using equation 5.10.

N = (0.5 * Sy) / TAUmax     % no units

N =

1.5573

6	Comparing these two results shows the more conservative nature of the maximum shear-stress theory, which gives a slightly lower safety factor.

7	Since the rod is a short beam, we need to check the shear due to transverse loading at point B on the neutral axis.  The maximum transverse shear stress at the neutral axis of a round rod was given in equation 4.15c.

Shear load at B

V = F                       % lbf

V =

1000

Cross-section area

A = (pi/4) * d^2            % in2

A =

1.7671

Shear stress at the neutral axis

TAUbending = (4*V)/(3*A)    % psi

TAUbending =

754.5123

Point B is in pure shear.  The total shear stress at point B is the algebraic sum of the transverse shear stress and the torsional shear stress, which both act on the same planes of the differential element in this case, in the same direction as shown in Figure 4-32c.

TAUmax = TAUxz + TAUbending % psi

TAUmax =

1.2827e+004

8	The safety factor for point B using the distortion-energy theory for pure shear (Eq. 5.9b) is

N = (0.577 * Sy) / TAUmax   % no units

N =

2.1143

and for the maximum shear-stress theory (Eq. 5.10) is

N = (0.5 * Sy) / TAUmax     % no units

N =

1.8321

Again, the latter is more conservative.