EXAMPLE 4-9

Combined Bending and Torsional Stresses

Problem	Find the most highly stressed locations on the bracket shown in Figure 4-30 and determine the applied and principal stresses at those locations.

Given	Applied load

F = 1000;    % lbf

Length of rod

l = 6;   % in

Length of arm

a = 8;   % in

Outside rod diameter

d = 1.5; % in

Assumptions	The load is static and the assembly is at room temperature.  Consider shear due to transverse loading as well as other stresses.

Solution	See Figures 4-30 to 4-33, M-Book Ex04-09.doc and M-File Ex04_09.m

1	We will limit our investigation to the rod, which is loaded in both bending (as a cantilever beam) and in torsion.  (The arm would also need to be analyzed for a complete design).  First, the load distributions over the rod's length need to be determined by drawing shear and moment diagrams for the rod.

2	The shear and moment diagrams will look similar to those for the cantilever beam in Example 4-5, the difference being that this force is at the end of the beam rather than at some intermediate point.  Figure 4-31 shows that the shear force is uniform across the beam length and its magnitude is equal to the applied load

Vmax = F    % lbf

Vmax =

1000

The maximum moment occurs at the wall and its magnitude is

Mmax = F*l  % lbf in

Mmax =

6000

(See Example 4-5 for derivations).

The torque applied to the rod is due to the force F acting at the end of the 8-in arm and is

Tmax = F*a  % lbf in

Tmax =

8000

Note that this torque is uniform over the length of the rod as it can only be reacted against by the wall.  Figure 4-31 shows all three of these loading functions.  It is clear from these graphs that the most heavily loaded cross section is at the wall where all three loads are maximum.

3	We will now take a section through the rod at the wall and examine the stress distributions within it due to these external loads.  Figure 4-32a shows the distribution across the section of the normal bending stresses, which are a maximum (+/-) at the outer fibers and zero at the neutral axis.  The shear stress due to transverse loading is a maximum at all points in the neutral (xz) plane and zero at the outer fibers (Figure 4-32b).

The shear stress due to torsion is proportional to the radius so is zero at the center (x axis) and a maximum at all points on the outer surface as shown in Figure 4-32c.  Note the differences between the distributions of the normal bending stress and the torsional shear stress.  The bending stresss magnitude is proportional to the distance y from the neutral plane and so is maximum at only the top and bottom of the section, whereas the torsional shear stress is maximal all around the perimeter.

4	We will choose two points, A and B of Figure 4-30, to analyze (also shown in Figure 4-32a) because they have the worst combination of stresses.  The largest tensile bending stress will be in the top outer fiber at point A and it combines with the largest torsional shear stress that is all around the outer circumference of the rod.  A differential element taken at point A is shown in Figure 4-33b.  Note that the normal stress (σx) acts on the x-face in the x-direction and the torsional shear stress (τxz) acts on the x-face in the positive z-direction.

At point B the torsional shear stress has the same magnitude as at point A, but the direction of the torsional shear stress (τxy) at point B is 90 deg (about the x-axis from the positive z to the negative y direction) different than at point A.  The shear stress due to transverse loading (τxy) is a maximum at point B.  Note that these shear stresses both act in the negative y-direction on the x-face at point B as shown in Figure 4-33c.  The transverse and torsional shear stresses then add at point B.

5	Find the normal bending stress and torsional shear stress on point A using equations 4.11b and 4.19a, respectively.

Moment of inertia

I = (pi * d^4) / 64     % in4

I =

0.2485

Distance to outer fiber

c = 0.5 * d     % in

c =

0.7500

Bending stress

SIGx = (Mmax * c) / I / 1000  % ksi (ksi = 1000 psi)

SIGx =

18.1083

Polar moment

J = 2 * I   % in4

J =

0.4970

Torsional shear stress

TAUxz = (Tmax * d) / (2 * J) / 1000    % ksi

TAUxz =

12.0722

6	Find the maximum shear stress and principal stresses that result from this combination of applied stresses using equations 4.6.

Normal stress in z-direction

SIGz = 0    % ksi

SIGz =

0

Maximum shear stress

TAUmax = sqrt(((SIGx - SIGz)/2)^2 + TAUxz^2)    % ksi

TAUmax =

15.0902

Principal stresses

SIG1 = (SIGx + SIGz) / 2 + TAUmax     % ksi

SIG2 = 0    % ksi

SIG3 = (SIGx + SIGz) / 2 - TAUmax     % ksi

SIG1 =

24.1444

SIG2 =

0

SIG3 =

-6.0361

7	Find the shear stress due to transverse loading at point B on the neutral axis.  The maximum transverse shear stress at the neutral axis of a round rod was given in equation 4.15c.

Cross section area

A = (pi * d^2) / 4      % in2

A =

1.7671

Transverse shear stress

TAUtran = (4 * Vmax) / (3 * A) / 1000      % ksi

TAUtran =

0.7545

Point B is in pure shear.  The total shear stress at point B is the algebraic sum of the transverse shear stress and the torsional shear stress, which both act on the same planes of the differential element.

Torsional shear stress

τtors=τxz=τxy

TAUtors = TAUxz     % ksi

TAUtors =

12.0722

Maximum shear stress

TAUmax = TAUtran + TAUtors      % ksi

TAUmax =

12.8267

which from equation 4.6 or the Mohr's circle can be shown to be equal to the largest principal stress for this point.

8	Point A has the larger principal stress in this case but note that the relative values of the applied torque and moment determine which of these two points will have the higher principal stress.  Both points must then be checked.