EXAMPLE 14-6

Fasteners in Eccentric Shear

Problem	Determine a size for the dowel pins in the bracket of Figure 14-41.

Given	Static force

P = 1200;    % lbf

Applied at… Distance to load

l = 5;       % in

Radius to pins

r = 1.5;     % in

Number of pins

n = 4;

Assumptions	All pins share load equally.  Use 40-48 HRC alloy steel for the pin material.

Solution	See Figures 14-41 and 14-42, M-Book Ex14-06.doc and M-File Ex14_06.m.

1	Calculate the moment of the applied force.

M = P * l   % lbf in

M =

6000

2	Calculate the magnitude of the force due to this moment at each pin.

FM = (M*r) / (n*r^2)    % lbf

FM =

1000

3	Find the amount of the direct force P felt at each pin.

FP = P / n  % lbf

FP =

300

4	Based on the vector diagram in Figure 14-42, pin B is the most heavily loaded and its resultant force is

FB = FP + FM    % lbf

FB =

1300

5	The shear stress in the pin will be τ=FBA=4⋅FBπ⋅d2 	(e)

and the factor of safety is found from Ns=Syτ where, from Table 14-11, 	(f)

Sy = 117*1000   % psi

Sy =

117000

6	Let the design factor of safety be Ns = 10 and eliminate τ from equations e and f to solve for a tentative pin diameter

Ns = 10;

d = sqrt((4*Ns*FB)/(pi*Sy))     % in

d =

0.3761

This is close to a standard size (0.375 in), which is readily available so let

d = 0.375;      % in