EXAMPLE 11-2

Designing a Compound Gear Train

Problem	Design a compound, spur-gear train for an overall train ratio of 29:1.

Given	Use 25 deg pressure angle gears with a module of 3 mm in all stages.

Rtrain = 29;

Assumptions	The largest ratio in any one gearset should be limited to about 10:1.  The minimum number of teeth on any pinion is 12 (from Table 11-4).

Rmax = 10;

Tmin = 12;

Solution	See M-Book Ex11-02.doc and M-File Ex11_02.m.

1	The required ratio is too large for one stage (one gearset), but two will each be within the 10:1 limitation.  We can get an idea of the gearset ratios needed by taking the square root of the desired train ratio: (29)0.5 = 5.385.  Thus, two gearsets with this ratio will do it.

Rapprox = sqrt(Rtrain)

Rapprox =

5.3852

2	Since the number of teeth on each gear must be an integer, see how close we can come to the 5.385:1 gearset ratio with integer combinations of teeth, starting with the smallest possible pinion:

12*Rapprox

13*Rapprox

14*Rapprox

ans =

64.6220

ans =

70.0071

ans =

75.3923

The second of these will be very close to the correct ratio when rounded to an integer.

3	Try two gearsets of 13t and 70t.  What will be the train ratio?

(70/13)*(70/13)

ans =

28.9941

4	If this is close enough for the application, the problem is solved.  The only situation in which it might not be acceptable would be where an exact ratio was required to provide a timing function.

5	Note that using two identical gearsets in a compound train automatically reverts it, allowing the input and output shafts to be concentric.