EXAMPLE 13-3

Design of a Helical Compression Spring for Static Loading

Problem	Design a compression spring for a static load over a known deflection.

Given	Minimum force

Finit = 100;     % lbf

Maximum force

Fwork = 150;     % lbf

Working deflection

DELTAy = 0.75;   % in

Assumptions	Use the least expensive, unpeened, cold-drawn spring wire (ASTM A227) since the loads are static.

Shear modulus

G = 11.5e6;      % psi

Solution	See M-Book Ex13-03.doc and M-File Ex13_03.m.

1	Assume a trial wire diameter from the available sizes in Table 13-2.

Wire diameter

d = 0.162;  % in

2	Assume a spring index of 8, which is in the middle of the recommended range, and calculate the mean coil diameter D from equation 13.5.

Spring index

C = 8;

Mean coil diameter

D = C*d     % in

D =

1.2960

3	Find the direct shear factor Ks and use it to calculate the shear stress in the coil at the larger force.

Direct shear factor

Ks = 1 + 0.5/C

Ks =

1.0625

Stress at Fwork

TAUwork = Ks * (8*Fwork*D) / (pi*d^3)   % psi

TAUwork =

1.2371e+005

4		Find the ultimate tensile strength of this wire material from equation 13.3 and Table 13-4 and use it to find the torsional yield strength from Table 13-6, assuming that the set has been removed and using the low end of the recommended range.

From Table 13-4, for A227 cold-drawn wire

A = 141.04 * 1000   % psi

b = -0.1822

A =

141040

b =

-0.1822

Ultimate tensile strength

Sut = A * d^b       % psi (assumes d [=] inches)

Sut =

1.9650e+005

Shear yield strength

Sys = 0.60 * Sut    % psi

Sys =

1.1790e+005

5	Find the safety factor against yielding at this working deflection from equation 13.14.

Safety factor at working deflection

Ns = Sys / TAUwork

Ns =

0.9530

6	Try increasing the wire diameter slightly, keeping the same spring index.  Recalculate the mean coil diameter, stress, strength, and safety factor.

Wire diameter

d = 0.192;  % slightly increased wire diameter

Spring index

C = 8;

Mean coil diameter

D = C*d     % in

D =

1.5360

Stress at Fwork

TAUwork = Ks * (8*Fwork*D) / (pi*d^3)   % psi

TAUwork =

8.8074e+004

Ultimate tensile strength

Sut = A * d^b       % psi (assumes d [=] inches)

Sut =

1.9051e+005

Shear yield strength

Sys = 0.60 * Sut    % psi

Sys =

1.1431e+005

Safety factor at working deflection

Ns = Sys / TAUwork

Ns =

1.2979

This appears to be acceptable, so we can go on to design the other spring parameters.

7	The spring rate is defined in this problem because of the two specified forces at a particular relative deflection.

Spring rate

k = (Fwork - Finit) / DELTAy    % lbf / in

k =

66.6667

8	To achieve this spring rate, the number of active coils must satisfy equation 13.7, solving for Na yields:

Number of active coils

Na = (d^4 * G) / (8 * D^3 * k)

Na =

8.0859

Na = 8  % rounding to nearest 1/4 coil

Na =

8

Note that we round it to the nearest 1/4 coil as the manufacturing tolerance cannot achieve better than that accuracy.  Having rounded the number of active coils, we must now calculate the spring rate using equation 13.7:

Corrected spring rate

k = (d^4 * G) / (8 * D^3 * Na)  % lbf / in

k =

67.3828

9	Assume squared and ground ends making the total number of coils, from Figure 13-9:

Total coils

Nt = Na + 2

Nt =

10

10	The shut height can now be determined.

Shut height

Ls = d*Nt   % in

Ls =

1.9200

11	The initial deflection to reach the smaller of the two loads is

Initial deflection

yinit = Finit / k

yinit =

1.4841

12	Assume a clash allowance of 15% of the working deflection:

Clash allowance

DELTAyClash = 0.15 * DELTAy  % in

DELTAyClash =

0.1125

13	The free length (see Figure 13-8) can now be found from

Lf = Ls + DELTAyClash + DELTAy + yinit   % in

Lf =

4.2666

14	The deflection to the shut height is

yshut = Lf - Ls     % in

yshut =

2.3466

15	The force at this shut height is

Fshut = k * yshut   % lbf

Fshut =

158.1177

16	The shut-height stress and safety factor are

Stress at Fshut

TAUshut = Ks * (8 * Fshut * D) / (pi * d^3)   % psi

TAUshut =

9.2840e+004

Safety factor at working deflection

Ns = Sys / TAUshut

Ns =

1.2312

which is acceptable.

17	To check for buckling, two ratios need to be calculated, Lf / D and ymax / Lf.

Slenderness ratio

sr = Lf / D

sr =

2.7777

Deflection ratio

yprm = (yinit + DELTAy) / Lf

yprm =

0.5236

Take these two values to Figure 13-14 and find that their coordinates are safely within the zones that are stable against buckling for either end-condition case.

18	The inside and outside coil diameters are

Inside coil dia

Di = D - d  % in

Di =

1.3440

Outside coil dia

Do = D + d  % in

Do =

1.7280

19	The smallest hole and largest pin that should be used with this spring are

Smallest hole

holeMin = Do + 0.05 * D     % in

holeMin =

1.8048

Largest pin

pinMax = Di - 0.05 * D      % in

pinMax =

1.2672

20	The total weight of the spring is

Weight density

RHO = 0.28;  % lbf / in3

Weight

Wt = (pi^2 * d^2 * D * Nt * RHO) / 4    % lbf

Wt =

0.3912

21	We now have a complete design specification for this A227-wire spring:

Wire diameter

d   % in

d =

0.1920

Outside diameter

Do  % in

Do =

1.7280

Total coils (ends squared and ground)

Nt  % coils

Nt =

10

Free length

Lf  % in

Lf =

4.2666