EXAMPLE 6-3

Determining Fatigue Stress Concentration Factors

Problem	A rectangular, stepped bar similar to that shown in Figure 4-36 is to be loaded in bending.  Determine the fatigue stress-concentration factor for the given dimensions.

Given	Using the nomenclature in Figure 4-36:

Tensile strength

Sut = 100;   % ksi

Dimensions

D = 2;       % in

d = 1.8;     % in

r = 0.25;    % in

Solution	See M-Book Ex06-03.doc and M-File Ex06_03.m.

1	The geometric stress concentration factor Kt is found from the equation in Figure 4-36:

Kt=A⋅rdb

where A and b are given in the same figure as a function of the D/d ratio, which is Dd=1.111 For this ratio, the coeffecients A and b are

A = 1.0147;

b = -0.2179;

giving

Kt = A * (r/d)^b

Kt =

1.5601

2	The notch sensitivity q of the material can be found by using the Neuber factor a from Figure 6-35 and Tables 6-6 to 6-8 in combination with equation 6.13, or by reading q directly from Figure 6-36. We will do the former. The Neuber factor from Table 6-6 for Sut = 100 ksi is 0.062. Note that this is the square root of a:

Neuber's factor

a = 0.062^2

a =

0.0038

Notch sensitivity

q = 1/(1+(sqrt(a)/sqrt(r)))

q =

0.8897

3	The fatigue stress concentration factor can now be found from equation 6.11b:

Kf = 1 + q * (Kt - 1)   % no units

Kf =

1.4983