EXAMPLE 14-4

Determining the Material Stiffness and Joint Constant

Problem	A pressure chamber is sealed by a gasketed cap fastened with eight preloaded bolts.  Find the material stiffness and joint constant for two designs of the assembly as shown in Figure 14-33, one with a confined gasket and one with an unconfined gasket.  Also determine the loads felt by bolts and material.

Given	The eight 3/8-16 UNC bolts are equispaced on the bolt circle.  The cylinder and joint dimensions are:

Cylinder inside diameter

Dp = 4.00;   % in

Bolt circle diameter

Dbc = 5.50;  % in

Outside flange diameter

Df = 7.25;   % in

Bolt diameter

d = 0.375;   % in

Number of bolts

Nbolts = 8;

Clamped length

lm = 1.50;   % in

Gasket thickness

t = 0.125;   % in

Cylinder pressure

p = 1500;    % psi

Assumptions	The gasket material is rubber.  Hardened, standard washers are used under the bolt heads and nuts.

Material properties:

Bolt

Eb = 30e6;   % psi

Members

Em = 30e6;   % psi

Gasket

Eg = 10e3;   % psi

Solution	See Figure 14-33, M-Book Ex14-04.doc and M-File Ex14_04.m.

1	Figure 14-33 shows two alternate designs of gasket for the joint on the same view to save space.  Whichever design is used, its gasket configuration will be present on both sides of the centerline.  Don't be confused by the depiction of different gaskets top and bottom; only one or the other will be used in the final assembly.  We will deal first with the confined gasket configuration.

2	The forces on the individual bolts can be found from the known pressure and cylinder dimensions assuming that all bolts share the load equally. The area of the end cap that is under pressure and the total force on the end cap is

A = (pi * Dp^2) / 4     % in2

A =

12.5664

Ptotal = p*A            % lnf

Ptotal =

1.8850e+004

and the applied force on each bolt is

P = Ptotal / Nbolts     % lbf

P =

2.3562e+003

3	First find the stiffness of one bolt.  Its tensile stress area is found from Table 14-1 to be

At = 0.0775;        % in2

The clamp length is given as 1.5 in.  A bolt with length

l = 2.00;           % in

will have sufficient protrusion to grip the nut.  The lengths of thread and shank of the bolt are then

lthd = 2*d + 0.25   % in

ls = l - lthd       % in

lthd =

1

ls =

1

from which we can find the length of thread lt that is in the clamp zone:

lt = lm - ls        % in

lt =

0.5000

4	Find the shank area and then the stiffness of the bolt from equation 14.11a.

Ab = (pi * d^2) / 4     % in2

kb = 1 / ((lt/(At * Eb)) + (ls/(Ab * Eb)))  % lbf / in

Ab =

0.1104

kb =

1.9348e+006

5	A confined gasket allows the metal surfaces to contact just as if there were no gasket present.  So the analysis of the material stiffness can ignore the confined gasket.  The material stiffness in the clamp zone around any one bolt with washers can be estimated with equation 14.17 using factors from Table 14-9a for steel.

A = 0.78715;

b = 0.62873;

6	The material stiffness from equation 14.17 is

km = d * Em * A * exp(b * d / lm)   % lbf / in

km =

1.0363e+007

7	The joint stiffness factor for the confined-gasket design from equation 14.13c is

C = kb / (km + kb)

C =

0.1573

8	The portions of the applied load P felt by the bolt and the material can now be found from equations 14.13.

Pb = C * P      % lbf

Pm = (1-C) * P  % lbf (eqn i)

Pb =

370.7005

Pm =

1.9855e+003

9	Now we will address the unconfined-gasket case.  The bolt stiffness is not affected by the gasket but the material stiffness is.  We now have two springs in series, the slightly shorter metal "barrel" of material and the gasket.  These combine according to equation 13.2b.  The length of the barrel shortened by the gasket thickness t is

lg = lm - t     % in

lg =

1.3750

10	The stiffness of this shorter thickness of clamped steel by itself is

km1 = d * Em * A * exp(b * d / lg)   % lbf / in

km1 =

1.0512e+007

11	The portion of the unconfined gasket subjected to the clamp force can be assumed to be from the outside diameter of the flange shown in Figure 14-33 to the inside diameter of the vessel.  The bolt hole should be subtracted from the gasket area.  The area of the clamped gasket around one bolt is

Ag = (pi/4) * (((Df^2 - Dp^2)/Nbolts) - d^2)    % in2

Ag =

3.4791

12	The stiffness of this piece of gasket is found from equation 14.11c.

km2 = (Ag * Eg) / t     % lbf / in

km2 =

2.7833e+005

The modulus of elasticity Eg of the gasket material is found in Table 14-9b.

13	The combined stiffness of the gasketed joint (from equation 13.2b) is

km = (km1 * km2) / (km1 + km2)  % lbf / in

km =

2.7115e+005

Note that the combined stiffness is essentially the same as that of the soft gasket alone since it dominates the equation.  We could have used the gasket stiffness kg to represent the joint stiffness km with little error.

14	The joint constant with the unconfined gasket is now

C = kb / (km + kb)

C =

0.8771

and

(1-C)

ans =

0.1229

15	The portions of the applied load P felt by the bolt and the material with a soft, unconfined gasket in the joint can now be found from equations 14.13.

Pb = C * P      % lbf

Pm = (1-C) * P  % lbf  (eqn p)

Pb =

2.0666e+003

Pm =

289.6188

16	See what has happened as a result of introducing an unconfined soft gasket. Compare equation i to p.  The bolt has gone from feeling only 16% of the applied load with no gasket (or a confined gasket) to feeling 88% with a soft, unconfined gasket.

In effect, the roles of the bolt and the material have been reversed by the intro- duction of the soft gasket.  The unconfined, soft gasket severely limits the ability of the bolt to accommodate fatigue loads, as was accomplished in the previous example.