EXAMPLE 9-7

Designing a Solid-Disk Flywheel

Problem	Design a suitable flywheel for the system of Example 9-6.   An overspeed safety factor of at least 2 is desired.

Given	An input torque-time function which varies over its cycle as shown in Figure 9-20.  The torque is varying during the 360 deg cycle about its average value and the energy variation per cycle is 26105 in-lb as shown in Table 9-5.

Flywheel energy variation per cycle

Ek = 26105;      % lbf in

Flywheel speed

OMEGAavg = 800;  % rad/sec

Assumptions	The one cycle of torque variation shown is representative of the steady-state condition.  The desired coefficient of fluctuation is 0.05. The system is in continuous operation with minimal start-stop cycles.  A steel of 62 kpsi yield strength will be used.  No keyway will be used in order to reduce stress concentrations. Instead, a tapered locking hub will frictionally couple it to the shaft and the hub will be axially bolted to the flywheel.

Coefficient of fluctuation

Cf = 0.05;

Yield strength

Sy = 65000;  % psi

Solution	See Figure 9-25, Table 9-6, M-Book Ex09-07.doc and M-File Ex09_07.m.

1	We know the amount of energy needed from the torque-time diagram from Example 9-6 and have defined the shaft ω and a desired coefficient of fluctuation.  From these data we can determine a required mass moment of inertia for the system Is using equation 9.22.

Is = Ek / (Cf * OMEGAavg^2) % lbf in sec2

Is =

0.8158

The flywheel need only supply a portion of this amount if other rotating masses such as the motor armature are present.  However, we will assume in this example that the flywheel will provide all of the required inertia making

Im = Is     % lbf in sec2

Im =

0.8158

2	The flywheel dimensions for this moment of inertia can be defined from equation 9.17d.  We will assume a steel material with

Weight density

GAMMA = 0.28;    % lbf / in3

Inside radius

ri = 1.000;      % in

Moment of inertia

Im=π2⋅γ2⋅ri4−ro4∗t=Is

So, the required Im can be obtained with an infinity of combinations of flywheel radius ro and thickness t for the assumed data.

3	The best solution to the moment of inertia equation will be one that balances the conflicting factors of flywheel size, weight, stresses, and safety factor.  Consider two possible designs, one with a small thickness t and the other with a large t.  The thin flywheel will be larger in diameter but considerably lighter than the thick one due to the nonlinearity of the terms involving ro.  But, as ro increases, so will the stresses because the mass at larger radius exerts more centrifugal force on the material.

4	To get a value of ro that is consistent with any desired safety factor, the tangential stress equation 9.23a can be used.  Equation 9.23a, evaluated at the inside radius where r = ri is

Imγ2⋅ϖ2⋅3+v4⋅1−v⋅ri2+ro2

A safety factor against overspeeding can be defined as

Nos=ϖyieldϖavg

where ωyield is the speed at which the flywheel will begin to yield.  Solving this equation for ωyield and substituting that into the equation of step 4 will allow us to solve for ro if we set the equation equal to the yield strength Sy.  Thus

σt=γg⋅Nos⋅ϖavg2⋅3+v4⋅1−v⋅ri2+ro2=Sy

Solving for ro for v = 0.28 and Nos = 2

v = 0.28;

Nos = 2;

g = 32.174 * 12 % in / sec2

g =

386.0880

ro = ((Sy - (GAMMA/g)*((3+v)/4)*(1-v)*(Nos*OMEGAavg)^2 * ri^2) /...

((GAMMA/g)*((3+v)/4)*(Nos*OMEGAavg)^2))^(1/2)

ro =

6.4789

5	We can now use equation for moment of inertia (step 2) to solve for t after first setting ro to an even fractional dimension.  (Smaller than that calculated above to increase the overspeeding safety factor).  Let ro = 6.375 in, then

ro = 6.375;  % in (standard size)

t = Im / ((pi/2)*(GAMMA/g)*(ro^4 - ri^4))   % in

t =

0.4338

Using ro = 6.375 in and t = 0.483 in (7/16 in), we can check the tangential stress at the inside diameter of the flywheel and the factors of safety against yielding and overspeeding.

ro = 6.375;  % in (standard size)

t = 0.438;   % in (7/16 in)

Tangential stress at

OMEGAavg    % rad/sec

OMEGAavg =

800

SIGt = (GAMMA/g)*OMEGAavg^2*((3+v)/4)*((1-v)*ri^2 + ro^2)   % psi

SIGt =

1.5742e+004

Factor of safety against yielding

Ny = Sy / SIGt

Ny =

4.1292

Flywheel speed at yielding

OMEGAyield = (Sy / ((GAMMA/g)*((3+v)/4)*...

((1-v)*ri^2 + ro^2)))^(1/2)    % rad/sec

OMEGAyield =

1.6256e+003

Factor of safety against overspeeding

Nos = OMEGAyield / OMEGAavg

Nos =

2.0320

The outside diameter and weight of the flywheel are

d = 2 * ro  % in

W = GAMMA * pi * (ro^2 - ri^2) * t  % lbf

d =

12.7500

W =

15.2729

6	To show the variation of parameters with flywheel geometry, this set of equations was solved for a list of possible thicknesses t chosen over a reasonable design range of 0.125 to 1.25 in.  Table 9-6 shows the resulting data and Figure E9-7 plots the trends.  Note that the weight increases as the outside diameter decreases and the thickness increases.  The maximum tangential stress at the inside radius decreases with decreasing ro, the safety factor against yielding increases from 2.1 to 6.7, and the safety factor for overspeed varies from 1.5 to 2.6 over this range of thicknesses.

Im  % (lbf in sec2) held constant

Im =

0.8158

t = 0.125:0.05:1.25;

ro = (Im * 2 * g ./ (pi * GAMMA * t) + ri^4).^0.25;

d = 2 * ro;  % in

SIGt = (GAMMA/g)*OMEGAavg^2*((3+v)/4)*...

((1-v)*ri^2 + ro.^2) / 1000;   % ksi

Ny = (Sy / 1000) ./ SIGt;

OMEGAyield = (Sy ./ ((GAMMA/g)*...

((3+v)/4)*((1-v)*ri^2 + ro.^2))).^(1/2);    % rad/sec

Nos = OMEGAyield ./ OMEGAavg;

W = GAMMA * pi .* (ro.^2 - ri^2) .* t;  % lbf

plot(t, W,'-.', t,d,':', t,SIGt,'--', t,Ny,'-', t,Nos,'--')

xlabel('thickness (in)')

legend('weight (lb)','diameter (in)','stress (ksi)','Ny','Nos')

Figure E9-7. Variation of weight, diameter, stress and safety factors with flywheel thickness.

7	The final design chosen is t = 0.438 in and ro= 6.375 in because it has a reasonable mix of parameter values (size, weight) and provides a safety factor of 2 against overspeeding.  In other words, the flywheel could be run at up to twice its design speed before it would yield.  The safety factor against yielding at the lower design speed will then always be higher and is now 4.1.  Choosing larger safety factors will exact a weight penalty, as can be seen in Figure 9-25.