EXAMPLE 4-5

Finding Beam Slope and Deflection of a Cantilever Beam Using Singularity Functions

Problem	Determine and plot the slope and deflection functions for the simply supported beam shown in Figure 4-22b.

Given	The load is the concentrated force shown.

Beam length

l = 10; % in

Load location

a = 4;  % in

Applied load

F = 400;  % lbf

Moment of inertia

I = 0.5;  % in4

Modulus of elasticity

E = 30e6;    % psi

Assumptions	Ignore the beam weight as negligible compared to the applied load.

Solution	See Figures 4-22b and 4-24, M-Book Ex04-05.doc and M-File Ex04_05.m

1	Write an equation for the load function in terms of equations 3.17 and integrate the resulting function twice using equations 3.18 to obtain the shear and moment functions.  Note use of the unit doublet function to represent the moment at the wall.  For the beam in Figure 4-22b,

q(x) = -M1<x - 0>-2 + R1<x - 0>-1 - F<x - a>-1	(a)

V(x) = -M1<x - 0>-1 + R1<x - 0>0 - F<x - a>0  + C1	(b)

M(x) =  -M1<x - 0>0 + R1<x - 0>1 - F<x - a>1 + C1x+ C2	(c)

θ(x) = ( -M1<x - 0>1 + R1<x - 0>2/2 - F<x - a>2/2 + C1x2/2  + C2x + C3) / EI	(d)

y(x) = ( -M1<x - 0>2/2 + R1<x - 0>3/6 - F<x - a>3/6 + C1x3/6 + C2x2/2 + C3x + C4) / EI	(e)

The reaction moment M1 at the wall is in the z direction and the forces R1 and F are in the y direction in equation (b).  All moments in equation (c) are in the z direction.

2	Because the reactions have been included in the loading function, the shear and moment diagrams both close to zero at each end of the beam, making C1 = C2 = 0.

3	The reaction force R1 and the reaction moment M1 can be calculated from equations (b) and (c) respectively by substituting the boundary conditions x = l+, V = 0, M = 0.  Note that we can substitute l for  l+ since their difference is vanishingly small.

V(l) = -M1<l - 0>-1 + R1<l - 0>0 - F<l - a>0   = 0	(f)

V = R1 – F = 0

R1 = F    % lbf

R1 =

400

M(l) = -M1<l - 0>0 + R1<l - 0>1 - F<l - a>1  = 0	    < cw>     (g)

M = -M1 + R1 l – F (l-a) = 0

M1 = R1*l - F*(l-a) % lbf in

M1 =

1600

Since w, l, and a are known from the given data, equation (f) can be solved for R1, and this result substituted into equation (g) to find M1.  Note that equation (f) is just ΣFy = 0, and equation (g) is ΣMz = 0.

4	Substitute x = 0, θ = 0 and x = 0,  y = 0 in equations d and e  and solve for C3 and C4.

θ(0) = ( -M1<0 - 0>1 + R1<0 - 0>2/2 - F<0 - a>2/2 + C3) / EI = 0

C3 =  M1<0 - 0>1 - R1<0 - 0>2/2 + F<0 - a>2/2	(h)

C3 = 0;  % lbf in2

y(0) = ( -M1<0 - 0>2/2 + R1<0 - 0>3/6 - F<0 - a>3/6 + C4) / EI = 0

C4 =  M1<0 - 0>2/2 - R1<0 - 0>3/6 + F<0 - a>3/6	(i)

C4 = 0;  % lbf in3

5	Substitution of the expressions for C3 ,C4, R1 and M1 from equations f, g, h, and i into equation e gives the deflection equation for the cantilever beam in Figure 4-22b.

y(x) = F( x3 - 3ax2 - <x - a>3) / 6EI	(j)

6	The maximum deflection of a cantilever beam is at its free end.  Substitute x = l in equation j to find ymax.

y(l) = F( l3 - 3al2 - <l - a>3) / 6EI = ymax	(k)

ymax = (F*a^2)/(6*E*I)*(a - 3*l)    % in

ymax =

-0.0018

7	The shear, moment, slope, and deflection diagrams can now be plotted with the same technique used in Examples 3-2B, 3-3B, and 3-4.

Range of x

x = 0:0.005*l:l;    % in

V = R1.*(x>=0).*(x-0).^0 - F.*(x>=a).*(x-a).^0;    % lbf

M = -M1.*(x>=0).*(x-0).^0 + R1.*(x>=0).*(x-0).^1 -...

F.*(x>=a).*(x-a).^1;    % lbf in

THETA = (1/(E*I))*(-M1.*(x>=0).*(x-0).^1 +...

(R1/2).*(x>=0).*(x-0).^2 -...

(F/2).*(x>=a).*(x-a).^2 + C3);

y = (1/(E*I))*((-M1/2).*(x>=0).*(x-0).^2 +...

(R1/6).*(x>=0).*(x-0).^3 -...

(F/6).*(x>=a).*(x-a).^3 + C3.*x + C4);

8	Graphs of the shear, moment, slope, and deflection functions are shown below.  Note that the beam slope becomes increasingly negative for the portion of the beam between the support and the load and then is constant to the right of the load.  While not very apparent at the small scale of the figure, the beam deflection is a straight line to the right of the point of application (x = 4 in) of the load.  If you wish, you may drag the graphs to other locations and enlarge them by dragging the lower left corner of the outline box that surrounds a graph after it has been selected.

subplot(2,2,1)

plot(x,V)

title('Shear Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Shear, V (lbf)')

ylim([-100 500])

subplot(2,2,2)

plot(x,M)

title('Moment Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Moment, M (lbf in)')

subplot(2,2,3)

plot(x,THETA)

title('Slope Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Slope (Rad)')

subplot(2,2,4)

plot(x,y)

title('Deflection Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Deflection (1000th in)')

FIGURE 4-24. Example 4-5 Graphs