EXAMPLE 13-4

Design of a Helical Compression Spring for Cyclic Loading

Problem	Design a compression spring for a dynamic load over a known deflection.

Given	Minimum force

Fmin = 60;       % lbf

Maximum force

Fmax = 150;      % lbf

Working deflection

DELTAy = 1.00;   % in

Forcing frequency

ff = 1000;       % cpm

Life (one shift)

L = 10;          % yr

Assumptions	Music wire (ASTM A228) will be used since the loads are dynamic.  Peening will be used to obtain a higher endurance strength.

Shear modulus

G = 11.5e6;     % psi

Solution	See Figures 13-18 and 13-19, M-Book Ex13-04.doc and M-File Ex13_04.m.

1	Find the the number of cycles that the spring will see over its design life.

Cycle life

Nlife = ff * 2080 * L * 60  % cycles

Nlife =

1.2480e+009

This large a number requires that an endurance limit for infinite life be used.

2	Find the the mean and alternating forces from equation 13.15a:

Alternating force

Fa = (Fmax - Fmin) / 2  % lbf

Fa =

45

Mean force

Fm = (Fmax + Fmin) / 2  % lbf

Fm =

105

3	Assume a trial wire diameter from the available sizes in Table 13-2.  Assume a spring index of 9 and calculate the mean coil diameter D from equation 13.5.

Wire diameter

d = 0.207   % in

d =

0.2070

Spring index

C = 9;       % coils

Mean coil diameter

D = C * d   % in

D =

1.8630

4	Find the direct shear factor Ks and use it to calculate the shear stress τi at the initial deflection (lowest defined force), and the mean stress τm.

Direct shear factor

Ks = 1 + 0.5 / C

Ks =

1.0556

Stress at Fmin

TAUi = Ks * (8 * Fmin * D) / (pi * d^3)   % psi

TAUi =

3.3875e+004

Stress at Fm

TAUm = Ks * (8 * Fm * D) / (pi * d^3)   % psi

TAUm =

5.9281e+004

5	Find the Wahl factor Kw and use it to calculate the alternating shear stress τa in the coil.

Wahl factor

Kw = (4*C - 1)/(4*C - 4) + 0.615 / C

Kw =

1.1621

Alternating stress

TAUa = Kw * (8 * Fa * D) / (pi * d^3)   % psi

TAUa =

2.7970e+004

6	Find the ultimate tensile strength of this wire material from equation 13.3 and Table 13-4 and use it to find the ultimate shear strength from equation 13.4 and the torsional yield strength from Table 13-6, assuming that the set has been removed and using the low end of the recommended range.

From Table 13-4, for A228 music wire

A = 184.649 * 1000; % psi

b = -0.1625;        % A and b from Table 13-4 for A228 music wire

Ultimate tensile strength

Sut = A*d^b         % psi (assumes d [=] in)

Sut =

2.3851e+005

Shear yield strength

Sys = 0.60 * Sut    % psi

Sys =

1.4310e+005

Ultimate shear strength

Sus = 0.67 * Sut    % psi

Sus =

1.5980e+005

7	Find the wire endurance limit for peened springs in repeated loading from equation 13.12 and convert it to fully reversed endurance strength with equation 13.16c.

Wire endurance limit

Sew = 67.5 * 1000   % psi

Sew =

67500

Fully reversed endurance limit

Ses = 0.5 * (Sew * Sus) / (Sus - 0.5*Sew)   % psi

Ses =

4.2787e+004

8	The safety factor is calculated from equation 13.16b.

Fatigue factor of safety

Nfs = (Ses*(Sus-TAUi)) / (Ses*(TAUm-TAUi)+Sus*TAUa)

Nfs =

0.9696

This is obviously not an acceptable design.  See Figure 13-18.  It shows the varia- tion of free length, outside diameter, safety factor, and total weight for the same wire diameter as that used above, but with a varying spring index, C.

If we decrease the spring index from 9 to 7, keeping all other parameters the same, we will obtain an acceptable design with Nfs = 1.3. Recalculating those results that depend on C:

Spring index

C = 7;       % coils

Mean coil diameter

D = C * d   % in

D =

1.4490

Direct shear factor

Ks = 1 + 0.5 / C

Ks =

1.0714

Stress at Fmin

TAUi = Ks * (8 * Fmin * D) / (pi * d^3)   % psi

TAUi =

2.6743e+004

Stress at Fm

TAUm = Ks * (8 * Fm * D) / (pi * d^3)   % psi

TAUm =

4.6800e+004

Wahl factor

Kw = (4*C - 1)/(4*C - 4) + 0.615 / C

Kw =

1.2129

Alternating stress

TAUa = Kw * (8 * Fa * D) / (pi * d^3)   % psi

TAUa =

2.2705e+004

Fatigue factor of safety

Nfs = (Ses*(Sus-TAUi)) / (Ses*(TAUm-TAUi)+Sus*TAUa)

Nfs =

1.2689

9	The spring rate is defined from the two specified forces at their relative deflection.

Spring rate

k = (Fmax - Fmin) / DELTAy   % lbf / in

k =

90

10	To get the defined spring rate, the number of active coils must satisfy equation 13.7, solving for Na yields:

Number of active coils

Na = (d^4 * G) / (8 * D^3 * k)

Na =

9.6392

Na = 9.75  % rounding to nearest 1/4 coil

Na =

9.7500

Note that we round it to the nearest 1/4 coil as the manufacturing tolerance cannot achieve better than that accuracy.  Having rounded the number of active coils, we must now calculate the spring rate using equation 13.7:

Corrected spring rate

k = (d^4 * G) / (8 * D^3 * Na)  % lbf / in

k =

88.9773

11	Assume squared and ground ends making the total number of coils, from Figure 13-9:

Nt = Na + 2

Nt =

11.7500

12	The shut height can now be determined.

Ls = d*Nt   % in

Ls =

2.4322

13	The initial deflection to reach the smaller of the two loads is

yinit = Fmin / k    % in

yinit =

0.6743

14	Assume a clash allowance of 15% of the working deflection:

DELTAyClash = 0.15 * DELTAy  % in

DELTAyClash =

0.1500

15	The free length (see Figure 13-8) can now be found from

Lf = Ls + DELTAyClash + DELTAy + yinit   % in

Lf =

4.2566

16	The deflection to the shut height is

yshut = Lf - Ls     % in

yshut =

1.8243

17	The force at this shut height is

Fshut = k * yshut   % lbf

Fshut =

162.3240

18	The shut-height stress and safety factor are

Stress at Fshut

TAUshut = Ks * (8 * Fshut * D) / (pi * d^3)   % psi

TAUshut =

7.2351e+004

Safety factor at working deflection

Ns = Sys / TAUshut

Ns =

1.9779

which is acceptable.

19	To check for buckling, two ratios need to be calculated,  Lf / D and ymax / Lf.

Slenderness ratio

sr = Lf / D

sr =

2.9376

Deflection ratio

yprm = (yinit + DELTAy) / Lf

yprm =

0.3934

Take these two values to Figure 13-14 and find that their coordinates are safely within the zones that are stable against buckling for either end-condition case.

20	The inside and outside coil diameters are

Inside coil diameter

Di = D - d  % in

Di =

1.2420

Outside coil diameter

Do = D + d  % in

Do =

1.6560

21	The weight of the spring's active coils is found from equation 13.11b and is

Weight density

RHO = 0.28;  % lbf / in3

Weight

Wa = (pi^2 * d^2 * D * Na * RHO) / 4    % lbf

Wa =

0.4182

22	The natural frequency of this spring is found from equation 13.11a and is:

Natural frequency

g = 32.174 * 12     % in / sec2

fn = 0.5 * sqrt((k*g)/Wa)       % Hz

fn = 0.5 * sqrt((k*g)/Wa) * 60  % cpm

g =

386.0880

fn =

143.3003

fn =

8.5980e+003

The ratio between the natural frequency and the forcing frequency is

Frequency ratio

FRatio = fn / ff

FRatio =

8.5980

which is sufficiently high.

23	We now have a complete design specification for this A228-wire spring:

Wire diameter

d   % in

d =

0.2070

Outside diameter

Do  % in

Do =

1.6560

Total coils (ends squared and ground)

Nt  % coils

Nt =

11.7500

Free length

Lf  % in

Lf =

4.2566