EXAMPLE 9-3

Designing a Stepped Shaft to Minimize Deflection

Problem	Design the same shaft as in Example 9-2 to have a maximum bending deflection of 0.002 in and a maximum angular deflection of 0.5 deg between sheave and gear.

Given	The loading is the same as in Example 9-2.  Shaft segment lengths:

A to B

lAB = 1.5;   % in

B to C

lBC = 3.5;   % in

C to D

lCD = 1.5;   % in

Tentative shaft diameters:

dAB = 0.875; % in

dBC = 0.750; % in

dCD = 0.669; % in

Peak torque

T = 146;     % lbf in

Modulus of rigidity (steel)

G = 12e6;    % psi

Assumptions	The lengths will remain the same as in the previous example, but diameters can be changed to stiffen the shaft if necessary.  The material is the same as in Example 9-2.

Solution	See Figures 9-11 to 9-13, M-Book Ex09-03.doc and M-File Ex09_03.m.

1	The torsional deflection is found from equations 9.9.  The polar area moments of inertia are first calculated for each segment of different diameter.

From A to B

JAB = (pi/32) * dAB^4   % in^4

JAB =

0.0575

From B to C

JBC = (pi/32) * dBC^4   % in^4

JBC =

0.0311

From C to D

JCD = (pi/32) * dCD^4   % in^4

JCD =

0.0197

Using them in equation 9.9c to find the angular deflection,

THETA = (T/G)*(lAB/JAB + lBC/JBC + lCD/JCD) % rad

THETAdeg = THETA * (360/(2*pi)) % deg

THETA =

0.0026

THETAdeg =

0.1499

This deflection is within the requested specification.

2	The moment function for this shaft was derived using singularity functions for each of two loading planes and then combined as equation k in Example 9-1.  It now must be divided by the product of E and the area moment of inertia I at each point along the shaft axis.  While E is constant, the value of I changes with each diametral change in the stepped shaft.

Total moment divided by EI

M(z)E⋅I=1E⋅I⋅Mx(z)2+My(z)212

Figure 9-11a shows the moment function for this shaft as derived in the previous examples and Figure 9-11b shows the M/EI function for the section diameters defined above and in Example 9-2.

3	The bending deflection is found by integrating the M/EI function twice.

θ=ME⋅I dz+C3δ=ME⋅I+C3 dz dz+C4

4	The first integration of the M/EI function from equation c gives the beam slope and the second integration gives the deflection function.  In previous discussions of beam deflection, (see Section 4.10 and Examples 4-4 to 4-7) the cross-section I of the beam was constant along the length.  In a stepped shaft, I is a function of the shaft length.  This makes the analytical integration of the M/EI function much more complicated.  A simpler approach is to numerically integrate the function twice using a trapezoidal or Simpson's rule.  See Example 9-9 and M-Book Ex09-09.doc for an example of using numerical integration on a shaft with multiple steps.

5	Since the shaft deflection is zero at z = 0, C4 = 0.  The other constant of integration C3 can be determined numerically (see Example 9-9).  Figure 9-12a shows the beam slope in the y-direction as integrated by a trapezoidal rule, and also shows the corrected slope function.  The integrated result is shifted up by an amount equal to the integration constant C3.  However, we do not know where the proper zero crossover is for this function, so cannot determine C3 from the beam-slope function.

6	The as-integrated deflection function in Figure 9-12b does not equal zero at the second support.  Since the deflection is really zero there, the difference in this integrated function and zero at the support can be used to determine the constant of integration C3.

A line is drawn in Figure 9-12b from the origin to the point on the curve at z = 5 where the function should be zero.  The slope of this straight line is the constant C3.

7	These deflection values are plotted in Figure 9-13 for the shaft diameters from Example 9-2.  The magnitude of the deflection at the gear is 0.0003 in, which is well within the requested specification.  At the sheave the deflection is 0.001 in, also within the specification.  The deflection at the right-hand end of the shaft is 0.002 in.