EXAMPLE 4-1

Determining Principal Stresses Using Mohr's Circles

Problem	A biaxial stress element as shown in Figure 4-2 has the given stress components.  Use Mohr's circles to determine the principal stresses. Check the result with a numerical method.

Given	Normal stresses

SIGx = 40;   % ksi    (ksi = 1000 x psi)

SIGy = -20;  % ksi

Shear stress (ccw)

TAUxy = 30;  % ksi ccw

Solution	See Figures 4-2 and 4-5, M-Book Ex04-01.doc and M-File Ex04_01.m

1	Construct the Mohr-plane axes as shown in Figure 4-5b and label them σ and τ.

2	Lay off the given applied stress σx (as line OA) to scale along the normal stress (horizontal) axis.  Note that σx is a tensile (positive) stress in this example.

3	Lay off the given applied stress σy (as line OB) to scale along the normal stress axis.  Note that σy is also a compressive (negative) stress in this example.

4	Figure 4-2 shows that the shear stresses τxy create a ccw couple on the element.  This couple is balanced for equilibrium by the cw couple provided by the shear stresses τyx.  Recall that both these shear stresses, τxy and τyx , are equal according to equation 4.2 and are positive according to the stress sign convention.  But, instead of using the stress sign convention, they are plotted on the Mohr circle according to the rotation that they imply to the element, using Mohr's left-handed sign convention of cw+ and ccw-.

5	Draw a vertical line downward (ccw-) from the tip of σx (as line AC) to represent the scaled magnitude of τxy.  Draw a vertical line upward (cw+) from the tip of σy (as line BD) to represent the scaled magnitude of τyx.

6	The diameter of one Mohr's circle is the distance from point C to point D.  Line AB bisects CD.  Draw the circle using this intersection as the center and the distance from the intersection to C or D as the radius.

Center at

SIGcent = (SIGx + SIGy) / 2 % ksi

SIGcent =

10

Radius

R = sqrt(((SIGx - SIGy) / 2)^2 + TAUxy^2)   % ksi

R =

42.4264

7	Two of the three principal normal stresses are then found at the two intersections that this Mohr's circle makes with the normal stress axis at points P1 and P3:

SIG1 = SIGcent + R  % ksi

SIG3 = SIGcent - R  % ksi

SIG1 =

52.4264

SIG3 =

-32.4264

8	Since there were no applied stresses in the z direction in this example, it is a 2-D stress state and the third principal stress, σ2, is known to be zero, thus is located at point O, also labeled P2

SIG2 = 0    % ksi

SIG2 =

0

9	There are still two other Mohr's circles to be drawn.  The three Mohr's circles are defined by the diameters (σ1 - σ3), (σ1 - σ2), and (σ2 - σ3), which in this case, are the lines P1P3, P1P2, and P2P3. The three circles are shown in Figure 4-5c.

10	Extend horizontal tangent lines from the top and bottom extremes of each Mohr's circle to intersect the shear (vertical) axis.  This determines the value of the principal shear stress associated with each pair of principal normal stresses:

TAU13 = (SIG1 - SIG3) / 2  % ksi

TAU12 = (SIG1 - SIG2) / 2  % ksi

TAU23 = (SIG2 - SIG3) / 2  % ksi

TAU13 =

42.4264

TAU12 =

26.2132

TAU23 =

16.2132

Note that despite having only two nonzero principal normal stresses, there are three nonzero principal shear stresses.  However, only the largest of these,

TAUmax = TAU13  % ksi

TAUmax =

42.4264

is of interest for design purposes.

11	We can also determine the angles (with respect to our original xyz axes) of the principal normal and principal shear stresses from the Mohr's circle.  These angles are only of academic interest if the material is homogeneous and isotropic.  If it is not isotropic, its material properties are direction-dependent and the directions of the principal stresses are then important.  The angle 2φ = -45 deg in Figure 4-5b represents the orientation of the principal normal stress with respect to the x axis of our original system.  Note that the line DC on the Mohr plane is the x axis in real space and the angles are measured according to Mohr's left- handed convention (cw+).  Since angles on the Mohr plane are double those in real space, the angle of the principal stess σ1 with respect to the real space x axis is φ = 22.5 deg cw.  The stress σ3 will be 90 deg from σ1 and the maximum shear stress τ13 will be 45 deg from the σ1 axis in real space.