EXAMPLE 11-1

Determining Gear Tooth and Gear Mesh Parameters

Problem	Find the gear ratio, circular pitch, base pitch, pitch diameters, pitch radii, center distance, addendum, dedendum, whole depth, clearance, outside diameters, and contact ratio of a gearset with the given parameters.  If the center distance is increased 2% what is the new pressure angle?

Given	A 6 pd, 20 deg pressure angle, 19-tooth pinion is meshed with a 37 tooth gear.

Diametral pitch

pd = 6;  % in-1

Number of teeth

Np = 19;

Ng = 37;

Pressure angle

PHIdeg = 20;    % deg

PHI = PHIdeg * (2*pi)/360   % rad

PHI =

0.3491

Assumptions	The tooth forms are standard AGMA full-depth involute profiles.

Solution	See M-Book Ex11-01.doc and M-File Ex11_01.m.

1	The gear ratio is easily found from the given tooth numbers on pinion and gear using equation 11.5b.

Gear ratio

mG = Ng / Np

mG =

1.9474

2	The circular pitch can be found either from equation 11.3a or 11.4b.

Circular pitch

pc = pi / pd    % in

pc =

0.5236

3	The base pitch measured on the base circle is (from equation 11.3b):

Base pitch

pb = pc * cos(PHI)  % in

pb =

0.4920

4	The pitch diameters and pitch radii of pinion and gear are found from equation 11.4a.

Pinion:

Pitch diameter

dp = Np / pd    % in

dp =

3.1667

Pitch radius

rp = 0.5 * dp   % in

rp =

1.5833

Gear:

Pitch diameter

dg = Ng / pd    % in

dg =

6.1667

Pitch radius

rg = 0.5 * dg   % in

rg =

3.0833

5	The nominal center distance C is the sum of the pitch radii:

C = rp + rg     % in

C =

4.6667

The center distance can also be found from

C = (Np + Ng) / (2 * pd)   % in

C =

4.6667

6	The addendum and dedendum are found from the equations in Table 11-1:

Addendum

a = 1.0 / pd    % in

a =

0.1667

Dedendum

b = 1.25 / pd   % in

b =

0.2083

7	The whole depth ht is the sum of the addendum and the dedendum.

ht = a + b      % in

ht =

0.3750

8	The clearance is the difference between the addendum and the dedendum.

c = b - a       % in

c =

0.0417

9	The outside diameter of each gear is the pitch diameter plus two addenda:

Outside diameter

Pinion

Dop = dp + 2*a  % in

Dop =

3.5000

Gear

Dog = dg + 2*a  % in

Dog =

6.5000

10	The contact ratio is found from equations 11.2 and 11.7.

Length of action

Z = sqrt((rp+a)^2 - (rp*cos(PHI))^2) +...

sqrt((rg+a)^2 - (rg*cos(PHI))^2)-C*sin(PHI)

Z =

0.7975

Contact ratio

mp = Z/pb

mp =

1.6209

11	If the center distance is increased from the nominal value due to assembly errors or other factors, the effective pitch radii will change by the same percentage.  The gears' base radii will remain the same.  The new pressure angle can be found from the changed geometry.  For a 2% in crease in center distance (1.02X):

Working pressure angle

PHInew = acos((rp * cos(PHI))/(1.02 * rp))  % rad

PHInewDeg = acos((rp * cos(PHI))/(1.02 * rp)) * 360/(2*pi)  % deg

PHInew =

0.3995

PHInewDeg =

22.8879