EXAMPLE 9-1

Shaft Design for Steady Torsion and Fully Reversed Bending

Problem	Design a shaft to support the attachments shown in Figure 9-5 with a minimum design safety factor of 2.5.

Given	Transmitted power

P = 2;          % hp

P = 2 * 396e3   % lbf in / s

P =

792000

Shaft speed

OMEGA = 1725;   % rpm

Shaft dimensions (see Fig.  9-5):

a = 1.5;        % in

b = 5.0;        % in

c = 6.5;        % in

p = 2.0;        % in

q = 6.75;       % in

s = 1.50;       % in

Pitch dia, gear

dg = 6.00;      % in

rg = 0.5*dg     % in

rg =

3

Pitch dia, sheave

ds = 6.00;      % in

rs = 0.5*ds     % in

rs =

3

Gear pressure angle

PHIdeg = 20;    % deg

PHI = PHIdeg * (2 * pi / 360)   % rad

PHI =

0.3491

Assumptions	There are no applied axial loads.  Steel will be used for infinite life.  Assume a stress concentration factor of 3.5 for the step radii in bending, 2 for step radii in torsion, and 4 at the keyways.  Since the torque is steady and the bending moment fully reversed, the ASME method of equation 9.6 can be used.

Solution	See M-Book Ex09-01.doc and M-File Ex09_01.m.

1	First determine the transmitted torque from the given power and angular velocity using equation 9.1.

T = P/(OMEGA * 2* pi)     % lbf in

T =

73.0729

This torque exists only over the portion of shaft between the sheave and the gear and is uniform in magnitude over that length as shown in Figure 9-6.

2	The tangential forces on the sheave and gear are found from the torque and their respective radii.  A V-belt has tension on both sides and the ratio between the force F1 on the tight side and F2 on the slack side is usually assumed to be about 5.  The net force associated with the driving torque is Fn = F1 - F2, but the force that bends the shaft is Fs = F1 + F2.  Combining these relationships gives (looking from the sheave end)

Tension ratio

RHO = 5     % no units

RHO =

5

Net belt force

Fn = T/rs   % lbf

F1 = Fn / (1 - (1/RHO)) % lbf

F2 = F1 - Fn    % lbf

Fn =

24.3576

F1 =

30.4470

F2 =

6.0894

Total belt force

Fs = F1 + F2    % lbf

Fs =

36.5364

3	The tangential and radial force at the spur-gear tooth is

Tangential

Fgt = T / rg    % lbf

Fgt =

24.3576

The spur gear has a 20 deg pressure angle as shown, which means that there will also be a radial component of force at the gear tooth of

Radial

Fgr = Fgt * tan(PHI)    % lbf

Fgr =

8.8655

4	We will consider the gear and sheave forces to be concentrated at their centers.  Solve for the reaction forces in the xz and yz planes with the trial shaft dimensions and the FBDs below.

FIGURE E9-1. Shaft load diagram, x-z plane

FIGURE E9-2. Shaft load diagram, y-z plane

Summing moments about A and forces, first in the xz-plane, then in the yz-plane.

ΣMA:		R2x⋅b+Fs⋅q+Fgr⋅p=0

R2x = (1/b)*(-Fgr*p - Fs*q)  % lbf

R2x =

-52.8704

ΣFx:		R1x+Fgr+R2x+Fs=0

R1x = -Fgr - R2x - Fs   % lbf

R1x =

7.4685

ΣMA:		R2y⋅b−Fgt⋅p=0

R2y = (1/b)*(Fgt*p) % lbf

R2y =

9.7431

ΣFy:		R1y−Fgt+R2y=0

R1y = Fgt - R2y     % lbf

R1y =

14.6146

5	The shear load and bending moment acting on the shaft can now be found.  Write an equation for the loading function q using singularity functions, integrate it to get the shear function V, and integrate again for the moment function.  Do this for each plane and then combine the moments in each plane to get the total moment as a function of z.

Range of z and singularity function

xz-plane:

q = R1x<z>-1 + Fgr<z - p>-1 + R2x<z - b>-1 + Fs<z - q>-1

V = R1x<z>0 + Fgr<z - p>0 + R2x<z - b>0 + Fs<z - q>0

My = R1x<z>1 + Fgr<z - p>1 + R2x<z - b>1 + Fs<z - q>1

Writing the equation for My in Matlab form and plotting,

z = 0:0.2:7;    % in

My = @(z) R1x .* (z>=0) .* z + Fgr .* (z>=p) .*...

(z-p) + R2x .* (z>=b) .* (z-b) + Fs .* (z>=q) .* (z-q);

plot(z,My(z))

xlabel('Distance along the z axis (in)')

ylabel('Moment in x-z plane (lbf in)')

FIGURE E9-3. Moment diagram, x-z plane

yz-plane:

q = R1y<z>-1 - Fgt<z - p>-1 + R2y<z - b>-1

V = R1y<z>0 - Fgt<z - p>0 + R2y<z - b>0

Mx = R1y<z>1 - Fgt<z - p>1 + R2y<z - b>1

Writing the equation for Mx in Matlab form and plotting,

Mx = @(z) R1y .* (z>=0) .* z - Fgt .* (z>=p) .*...

(z-p) + R2y .* (z>=b) .* (z-b);

plot(z,Mx(z))

xlabel('Distance along the z axis (in)')

ylabel('Moment in y-z plane (lbf in)')

FIGURE E9-4. Moment diagram, y-z plane

The equation for the total moment as a function of z is the sqaure-root of the sum of the squares of the moments in the x and y directions:

Total moment

M = @(z)(Mx(z).^2 + My(z).^2).^(1/2);

plot(z,M(z))

xlabel('Distance along the z axis (in)')

ylabel('Total Moment magnitude (lbf in)')

FIGURE E9-5. Total moment diagram, both planes combined

6	The shear and moment distributions over the shaft length are shown above.  The applied torque is uniform over the portion of the shaft between points B and D as shown in Figure 9-6.  Within that length, there are three locations of concern where a moment occurs in combination with a stress concentration, point B at the step and keyway under the gear, point C at the right bearing where there is a step with a small radius to fit the bearing, and point D at the sheave step.  Note that because of its high stress concentration, the snap-ring groove used for axial location has been placed at the end of the shaft where the moment and torque are both zero.

Moments at B, C, and D are:

At B:

z = p       % in

MB = M(z)   % lbf in

z =

2

MB =

32.8246

At C:

z = b       % in

MC = M(z)   % lbf in

z =

5

MC =

63.9388

At D:

z = c       % in

MD = M(z)   % lbf in

z =

6.5000

MD =

9.1341

7	A trial material needs to be selected for the computations.  We will first try an inexpensive, low-carbon, cold-rolled steel such as SAE 1020 with properties given below.  Though not exceptionally strong, this material has low notch sensitivity, which will be an advantage given the large stress concentrations.  Calculate the uncorrected endurance strength using equation 6.5:

Tensile strength

Sut = 65;   % ksi

Yield strength

Sy = 38;    % ksi

Uncorrected endurance limit

Sprme = 0.5 * Sut   % ksi

Sprme =

32.5000

Endurance limit correction factors:

Load

Cload = 1;

Size

Csize = @(d) 0.869 * d^-0.097;  % no units (assumes d [=] inches)

Surface (machined)

Csurf = 2.7 * Sut^-0.265        % no units (assumes Sut [=] ksi)

Csurf =

0.8932

Temperature

Ctemp = 1;

Reliability (50%)

Creliab = 1;

Modified endurance limit (as a function of the unknown diameter, d):

Se = @(d) Cload * Csize(d) * Csurf *...

Ctemp * Creliab * Sprme*1000;

8	The notch sensitivity is found, for an assumed notch radius of r = 0.01 in, from equation 6.13 as

Bending

a = 0.100^2     % in

r = 0.010;      % in

q = 1 / (1 + sqrt(a/r))

a =

0.0100

q =

0.5000

Torsion

a = 0.075^2     % in

r = 0.010;      % in

qs = 1 / (1 + sqrt(a/r))

a =

0.0056

qs =

0.5714

9	The fatigue stress-concentration factor is found from equation 6.11b using the assumed geometric stress-concentration factors noted below.

Bending at a step

Ktb = 3.5;

Torsion at a step

Kts = 2.0;

Keyway

Ktk = 4.0;

At C, with reversed bending and steady torsion:

Kfb = 1 + q * (Ktb - 1)

Kfs = 1 + qs * (Kts - 1)

Kfb =

2.2500

Kfs =

1.5714

From equation 6.17 we find in this case, the same factor should be used on the mean torsional stress component, thus:

Kfsm = Kfs

Kfsm =

1.5714

10	The shaft diameter at point C can now be found from equation 9.6 using the moment magnitude at that point as given above.  Since Se is a function of the unknown diameter d, we must use iteration to solve this equation.

Design factor of safety

Nf = 2.5

Nf =

2.5000

d=32⋅Nfπ⋅Kfb⋅MCSed2+34⋅Kfsm⋅TSy21213

This equation cannot be solved directly because the right-hand-side contains a function of d.  Although it can be set up to be solved using Matlab's fzero function, we will perform the necessary iteration manually.  The right-hand side (RHS) of this equation is

RHS = @(d) (((32*Nf)/pi)*((Kfb*(MC/Se(d)))^2 +...

(3/4)*(Kfsm*(T/(Sy*1000)))^2)^(1/2))^(1/3);

Starting the iteration with a guess

d = 0.5     % in

d =

0.5000

RHS(d)

ans =

0.5328

d=RHS(d)

d =

0.5328

RHS(d)

ans =

0.5337

d=RHS(d)

d =

0.5337

RHS(d)

ans =

0.5337

For this value of d,

d

Csize(d)

Se(d)   % psi

d =

0.5337

ans =

0.9236

ans =

2.6810e+004

The next larger standard ball bearing diameter is 0.554 in, so let

d2 = 0.544   % in

d2 =

0.5440

If Kfsm is set to 1 as ASME recommends, then equation 9.6 gives d = 0.520 in.  If the more general equation 9.8 is used, the result is d = 0.557 in.  Note that the ASME method is less conservative than equation 9.8 as it gives smaller shaft diameters for the same safety factor.  A modified Goodman diagram for this stress element is shown in Figure 9-8b.  It predicts failure from fatigue.

11	At point B, under the gear, the moment is less, but the stress-concentration factor is greater so it should be checked.  The bending- and torsion-fatigue stress-concentration factors at B are

Kfb = 1 + q*(Ktk - 1)

Kfs = 1 + qs*(Ktk - 1)

Kfsm = Kfs

Kfb =

2.5000

Kfs =

2.7143

Kfsm =

2.7143

12	The minimum recommended diameter at point B from equation 9.6 is

d=32⋅Nfπ⋅Kfb⋅MBSed2+34⋅Kfsm⋅TSy21213

The right-hand side (RHS) of this equation is

RHS = @(d) (((32*Nf)/pi)*((Kfb*(MB/Se(d)))^2 +...

(3/4)*(Kfsm*(T/(Sy*1000)))^2)^(1/2))^(1/3);

Starting the iteration with a guess

d = 0.5     % in

d =

0.5000

RHS(d)

ans =

0.5177

d=RHS(d)

d =

0.5177

RHS(d)

ans =

0.5179

For this value of d,

d

Csize(d)

Se(d)   % psi

d =

0.5177

ans =

0.9263

ans =

2.6889e+004

If Kfsm is set to 1 as ASME recommends, then equation 9.6 gives d = 0.444 in.  If the more general equation 9.8 is used, the result is d = 0.524 in.  A modified Goodman diagram for this stress element is shown in Figure 9-8a.  It predicts failure from fatigue.

Since d1 must be larger than d2 to provide the shoulder at C, let

d1 = 0.625  % in

d1 =

0.6250

13	Another location of possible failure is the step against which the sheave seats, at point D.  The moment is lower than at C, being about 9.1 lb-in.  (See Figure 9-7.)  However, the shaft will be stepped smaller there and will have the same order of stress concentration as at point C.  (The keyway for the sheave is in a region of zero moment and so is ignored.)  Using those data in equations 9.6 for point D:

Kfb = 1 + q*(Ktb - 1)

Kfs = 1 + qs*(Kts - 1)

Kfsm = Kfs

Kfb =

2.2500

Kfs =

1.5714

Kfsm =

1.5714

d=32⋅Nfπ⋅Kfb⋅MDSed2+34⋅Kfsm⋅TSy21213

The right-hand side (RHS) of this equation is

RHS = @(d) (((32*Nf)/pi)*((Kfb*(MD/Se(d)))^2 +...

(3/4)*(Kfsm*(T/(Sy*1000)))^2)^(1/2))^(1/3);

Starting the iteration with a guess

d = 0.5     % in

d =

0.5000

RHS(d)

ans =

0.4110

d=RHS(d)

d =

0.4110

RHS(d)

ans =

0.4108

For this value of d,

d

Csize(d)

Se(d)   % psi

d =

0.4110

ans =

0.9473

ans =

2.7498e+004

Choosing a standard diameter that still provides for a shoulder at D, let:

d3 = 0.500  % in

d3 =

0.5000

If Kfsm is set to 1 as ASME recommends, then equation 9.6 gives d = 0.360 in.  If the more general equation 9.8 is used, the result is d = 0.381 in.  A modified Goodman diagram for this stress element is shown in Figure 9-8a.  It predicts failure from yeilding.