EXAMPLE 7-4

Stresses in Combined Rolling and Sliding of Cylinders

Problem	A pair of calendering rolls are run together with a combination of rolling and sliding.  Find the maximum tensile, compressive, and shear stresses in the rollers.

Given	Roller radii

R1 = 1.250;  % in

R2 = 2.500;  % in

Radial load

F = 5000;    % lbf

Length of contact

L = 24;      % in

The parts are hardened steel.  Therefore

E = 30e6;    % psi

v = 0.28;

Assumptions	Coefficient of friction

MU = 0.33

MU =

0.3300

Solution	See M-Book Ex07-04.doc and M-File Ex07_04.m.

1	The contact-patch geometry is found in the same way as was done in Example 7-2.  Find the material constants from equation 7.9a.

Material constants

m1 = (1-v^2)/E  % psi^-1

m2 = m1         % psi^-1

m1 =

3.0720e-008

m2 =

3.0720e-008

The geometry constant is found from equation 7.15a.

Geometry constant

B = (1/2) * ((1/R1) + (1/R2))   % in^-1

B =

0.6000

and the patch half-width from equation 7.15b.

Contact patch half-width

a = ((2/pi)*(((m1+m2)/B)*(F/L)))^(1/2)    % in

a =

0.0037

where a is the half-width of the contact patch.  The rectangular contact-patch area is then

Contact area

A = 2 * L * a    % in2

A =

0.1769

2	The average and maximum contact pressure can now be found from equations 7.14b and c.

Average pressure

Pavg = F/A/1000              % ksi

Pavg =

28.2657

Maximum pressure

Pmax = (2*F)/(pi*a*L)/1000   % ksi

Pmax =

35.9890

The tangential pressure is found from equation 7.22f.

Tangential pressure

fmax = MU * Pmax             % ksi

fmax =

11.8764

3	With µ = 0.33, the principal stresses in the contact zone will be maximal on the surface (z = 0) at x = 0.3a from the centerline as shown in Figures 7-20 and 7-22.  The applied-stress components are found from equation 7.23a for the normal force and equation 7.23b for the tangential force.

For

x = 0.3*a

x =

0.0011

SIGxn = -Pmax * sqrt(1-(x^2 / a^2))     % ksi

SIGxt = -2 * fmax * (x/a)               % ksi

SIGzn = -Pmax * sqrt(1-(x^2 / a^2))     % ksi

SIGzt = 0                               % ksi

SIGxn =

-34.3313

SIGxt =

-7.1258

SIGzn =

-34.3313

SIGzt =

0

TAUxzn = 0                              % ksi

TAUxzt = -fmax * sqrt(1-(x^2 / a^2))    % ksi

TAUxzn =

0

TAUxzt =

-11.3293

4	Equations 7.24a and 7.24b can now be solved for the total applied stresses along the x, y, and z axes.

SIGx = SIGxn + SIGxt        % ksi

SIGz = SIGzn + SIGzt        % ksi

SIGx =

-41.4571

SIGz =

-34.3313

TAUxz = TAUxzn + TAUxzt     % ksi

TAUxz =

-11.3293

5	Since the rollers are long, we expect a plane strain condition to exist. The stress in the third dimension is found from equation 7.23b:

SIGy = v * (SIGx + SIGz)    % ksi

SIGy =

-21.2208

6	Unlike the pure-rolling case, these stresses are not principal because of the applied shear stress.  The principal stresses are found from equation 4.4 using a cubic root finding solution.

TAUxy = 0       % ksi

TAUyz = 0       % ksi

TAUzx = TAUxz   % ksi

TAUxy =

0

TAUyz =

0

TAUzx =

-11.3293

C2 = (SIGx + SIGy + SIGz)   % no units (assumes SIG [=] ksi)

C2 =

-97.0092

C11 = [SIGx TAUxy; TAUxy SIGy]

C12 = [SIGx TAUzx; TAUzx SIGz]

C13 = [SIGy TAUyz; TAUyz SIGz]

C11 =

-41.4571         0

0  -21.2208

C12 =

-41.4571  -11.3293

-11.3293  -34.3313

C13 =

-21.2208         0

0  -34.3313

C1 = det(C11) + det(C12) + det(C13)  % no units (assumes ksi)

C1 =

2.9032e+003

C0 = det([SIGx TAUxy TAUzx;

TAUxy SIGy TAUyz;

TAUzx TAUyz SIGz])    % no units (assumes ksi)

C0 =

-2.7479e+004

v = [1 -C2 C1 -C0]

v =

1.0e+004 *

0.0001    0.0097    0.2903    2.7479

s = roots(v)

s =

-49.7706

-26.0178

-21.2208

Principal stresses:

SIG1 = s(3)     % ksi

SIG2 = s(2)     % ksi

SIG3 = s(1)     % ksi

SIG1 =

-21.2208

SIG2 =

-26.0178

SIG3 =

-49.7706

The maximum shear stress is found from the principal stresses using equation 4.5.

TAU13 = (SIG1 - SIG3) / 2   % ksi

TAU13 =

14.2749

7	The principal stresses are maximum at the surface as seen in Figures 7-20 and 7-22.