EXAMPLE 6-6

Multiaxial Fluctuating Stresses

Problem	Determine the safety factors for the bracket tube shown in Figure 5-7.

Given	The material is 2024-T4 aluminum

Yield strength

Sy = 47;        % ksi

Tensile strength

Sut = 68;       % ksi

Tube length

l = 6;          % in

Arm length

a = 8;          % in

Tube OD

od = 2.0;       % in

Tube ID

id = 1.5;       % in

Load

Fmin = -200;    % lbf

Fmax = 340;     % lbf

Assumptions	The load is dynamic and the assembly is at room temperature.  Consider shear due to transverse loading as well as other stresses.  A finite life design will be sought with a life of N = 6.107 cycles.  The notch radius at the wall is r = 0.25 in and stress-concentration factors are for bending Kt = 1.7, and for shear, Kts = 1.35.

N = 6e7;     % cycles

r = 0.25;    % in

Kt = 1.7;

Kts = 1.35;

Solution	See Figure 5-7, M-Book Ex06-06.doc and M-File Ex06_06.m.  Also see Example 4-9 for a more complete explanation of the stress analysis for this problem.

1	Aluminum does not have an endurance limit.  Its endurance strength at 5E8 cycles can be estimated from equation 6.5c.  Since the Sut is larger than 48 ksi, the uncorrected S'f@5E8 is

Sprmf5E8 = 19;   % ksi

2	The correction factors are calculated from equations 6.7 and used to find a corrected endurance strength at the standard 5E8 cycles.

Cload = 1;

A95 = 0.0105 * od^2  				% in2

A95 =

0.0420

deq = sqrt(A95 / 0.0766)   	% in

deq =

0.7405

Csize = 0.869 * deq^-0.097  	% assumes deq [=] inches

Csize =

0.8947

Table 6-3 constants

A = 2.7;

b = -0.265;

Csurf = A * Sut^b   				% assumes Sut [=] ksi

Csurf =

0.8826

Ctemp = 1;

Creliab = 0.753;    				% for R = 99.9%

Sf5E8 = Cload * Csize * Csurf * Ctemp * Creliab * Sprmf5E8   % ksi

Sf5E8 =

11.2973

Note that the bending value of Cload is used despite the fact that there is both bending and torsion present.  The torsional shear stress will be converted to an equivalent tensile stress with the von Mises calculation.  Csurf is calculated from equation 6.7e using data from Table 6-3.  This corrected fatigue strength is still at the tested number of cycles, N = 5E8.

3	This problem calls for a life of 6E7 cycles, so a strength value at that life must be estimated from the S-N line of Figure 6-33b using the corrected fatigue strength at that life.  Equation 6.10a for this line can be solved for the desired strength after we compute the values of its coefficients a and b from equation 6.10c.

Sm = 0.90 * Sut

Sm =

61.2000

z = 5.699;          % From Table 6-5 for 5E8

b = (-1/z) * log10(Sm / Sf5E8)

b =

-0.1288

a = Sm / 10^(3*b)   % ksi

a =

148.9421

Sn = a * N^b        % ksi

Sn =

14.8436

Note that Sm is calculated as 90% of Sut because loading is bending rather than axial (see Eq. 6.9).  The value of z is taken from Table 6-5 for N = 5E8 cycles.  This is a corrected fatigue strength for the shorter life required in this case and so is larger than the corrected test value, which was calculated at a longer life.

4	The notch sensitivity of the material must be found to calculate the fatigue stress-concentration factors.  Table 6-8 shows the Neuber factors for hardened aluminum.  Interpolation gives a value of α = 0.1472 in at the material's Sut.  Equation 6.13 gives the resulting notch sensitivity for the assumed notch radius.

a = 0.147^2     % in (redefines 'a')

a =

0.0216

q = 1 / (1 + sqrt(a) / sqrt(r))

q =

0.7728

5	The fatigue stress-concentration factors are found from equation 6.11b using the given geometric stress-concentration factors for bending and torsion, respectively.

Kf = 1 + q * (Kt - 1)

Kfs = 1 + q * (Kts - 1)

Kf =

1.5410

Kfs =

1.2705

6	The bracket tube is loaded in both bending (as a cantilever beam) and in torsion.  The shapes of the shear, moment and torque distributions are shown in Figure 4-30.  All are maximum at the wall.  The alternating and mean components of the applied force, moment, and torque at the walls are

Loads

Fa = (Fmax - Fmin) / 2  % lbf

Fm = (Fmax + Fmin) / 2  % lbf

Fa =

270

Fm =

70

Moments

Ma = Fa * l     % lbf in

Mm = Fm * l     % lbf in

Mmax = Ma + Mm    % lbf in

Ma =

1620

Mm =

420

Mmax =

2040

Torques

a = 8.0;     % in (restores definintion of 'a')

Ta = Fa * a     % lbf in

Tm = Fm * a     % lbf in

Ta =

2160

Tm =

560

7	The fatigue stress-concentration factor for the mean stresses depends on the relationship between the maximum local stress in the notch and the yield strength as defined in equation 6.17, a portion of which is shown here.

Outer fiber

c = 0.5 * od    % in

c =

1

Moment of inertia

I = (pi/64) * (od^4 - id^4)        % in4

I =

0.5369

J = 2 * I       % in4

J =

1.0738

If Kf⋅σmax<Sy then Kfm = Kf and Kfsm = Kfs.

Kf * abs((Mmax * c) / I) / 1000    % ksi

ans =

5.8551

Since this ans is less than Sy = 47 ksi,

Kfm = Kf

Kfsm = Kfs

Kfm =

1.5410

Kfsm =

1.2705

In this case, there is no reduction in stress-concentration factors for the mean stress because there is no yielding at the notch to relieve the stress concentration.

8	The largest tensile bending stress will be in the top or bottom outer fiber at points A or A'.  The largest torsional shear stress will be all around the outer circumference of the tube.  (See Example 4-9 for more details.)  First take a differential element at point A or A' where both of these sresses combine.  (See Figure 4-32.)  Find the alternating and mean components of the normal bending stress and of the torsional shear stress on point A using equations 4.11b and 4.24b, respectively.

SIGa = Kf * (Ma * c) / I / 1000      % ksi

TAUa = Kfs * (Ta * c ) / J / 1000    % ksi

SIGm = Kfm * (Mm * c) / I / 1000     % ksi

TAUm = Kfsm * (Tm * c ) / J / 1000   % ksi

SIGa =

4.6496

TAUa =

2.5557

SIGm =

1.2055

TAUm =

0.6626

9	Find the alternating and mean von Mises effective stresses at point A from equation 6.22b.

SIGxa = SIGa    % ksi

SIGya = 0;      % ksi

TAUxya = TAUa   % ksi

SIGprimeA = sqrt(SIGxa^2 + SIGya^2 - SIGxa*SIGya + 3 * TAUxya^2)    % ksi

SIGxa =

4.6496

TAUxya =

2.5557

SIGprimeA =

6.4198

SIGxm = SIGm    % ksi

SIGym = 0;      % ksi

TAUxym = TAUm   % ksi

SIGprimeM = sqrt(SIGxm^2 + SIGym^2 - SIGxm*SIGym + 3 * TAUxym^2)    % ksi

SIGxm =

1.2055

TAUxym =

0.6626

SIGprimeM =

1.6644

10	Because the moment and torque are both caused by the same applied force, they are synchronous and in-phase and any change in them will be in a constant ratio.

This is a Case 3 situation and the safety factor is found using equation 6.18e.

Nf = (Sn * Sut) / (SIGprimeA * Sut + SIGprimeM * Sn)

Nf =

2.1883

11	Since the tube is a short beam, we need to check the shear due to transverse loading at point B on the neutral axis where the torsional shear is also maximal.  The maximum transverse shear stress at the neutral axis of a hollow, thin-walled, round tube was given as equation 4.15d.

Cross-section area

A = (pi/4) * (od^2 - id^2)    % in2

A =

1.3744

TAUabend = Kfs * (2 * Fa) / A / 1000   % ksi

TAUmbend = Kfsm * (2 * Fm) / A / 1000  % ksi

TAUabend =

0.4992

TAUmbend =

0.1294

Point B is in pure shear.  The total shear stress at point B  is the sum of the transverse shear stress and the torsional shear stress which act on the same planes of the element.

TAUatotal = TAUabend + TAUa    % ksi

TAUmtotal = TAUmbend + TAUm    % ksi

TAUatotal =

3.0548

TAUmtotal =

0.7920

12	Find the alternating and mean von Mises effective stresses at point B from equation 6.22b.

SIGxa = 0;           % ksi

SIGya = 0;           % ksi

TAUxya = TAUatotal   % ksi

SIGprimeA = sqrt(SIGxa^2 + SIGya^2 - SIGxa*SIGya + 3 * TAUxya^2)    % ksi

TAUxya =

3.0548

SIGprimeA =

5.2911

SIGxm = 0;           % ksi

SIGym = 0;           % ksi

TAUxym = TAUmtotal   % ksi

SIGprimeM = sqrt(SIGxm^2 + SIGym^2 - SIGxm*SIGym + 3 * TAUxym^2)    % ksi

TAUxym =

0.7920

SIGprimeM =

1.3718

13	The safety factor for point B is found using equation 6.18e.

Nf = (Sn * Sut) / (SIGprimeA * Sut + SIGprimeM * Sn)

Nf =

2.6551

Both points A and B are safe against fatigue failure.