EXAMPLE 14-5

Determining the Torque to Generate a Bolt Preload

Problem	Find the torque required to preload the bolt in Example 14-3.

Given	A 5/16-18 UNC-2A, grade 5.2 bolt, 2.5 in long, preloaded to 90% of proof strength with an axial force of 4011 lb.

Diameter

d = 0.3125;  % in

Preload

Fi = 4011;   % lbf

Assumptions	The threads will be lubricated.  Assume a coefficient of friction of 0.15 and a torque coefficient of 0.21.

Solution	See M-Book Ex14-05.doc and M-File Ex14_05.m.

1	The required tightening torque can be estimated with equation 14.18d:

Ti = 0.21 * Fi * d  % lbf in

Ti =

263.2219