EXAMPLE 15-3

Design of a Long-Shoe Drum Brake

Problem	For the drum brake arrangement shown in Figure 15-9, determine the friction torque T, application force Fa, and reaction forces Rx, Ry.

Given	The dimensions are

a = 180/1000;   % m

b = 90/1000;    % m

r = 100/1000;   % m

w = 30/1000;    % m

THETA1deg = 30;             % deg

THETA1 = 30 * (2*pi/360)    % rad

THETA1 =

0.5236

THETA2deg = 120;            % deg

THETA2 = 120 * (2*pi/360)   % rad

THETA2 =

2.0944

Assumptions	The brake is self-energizing.

Coefficient of friction

MU = 0.35;

Maximum lining pressure

pmax = 1.5e6;   % Pa

Solution	See Figure 15-9 and M-Book Ex15-03.doc and M-File Ex15_03.m.

1	The maximum pressure occurs at θ = 90 deg not at θ2.  Thus, in the equations where pmax is divided by sin θ, we will use 90 deg instead of 120 deg.  The given angles θ1 and θ2 were converted to radians θ1=0.524 rad, θ2=2.094 rad.

Angle at which p is a maximum:

THETAmdeg = 90;             % deg

THETAm = 90 * (2*pi/360)    % rad

THETAm =

1.5708

2	Calculate the moment MFn about O due to the normal force using equation 15.14a.

MFn = w * r * b * (pmax / sin(THETAm)) * ...

((1/2)*(THETA2 - THETA1) -...

(1/4)*(sin(2*THETA2) - sin(2*THETA1)))   % N m

MFn =

493.4564

3	Calculate the moment MFf about O due to the friction force using equation 15.14b.

MFf = -MU * w * r * (pmax / sin(THETAm)) * ...

(r * (cos(THETA2) - cos(THETA1)) + ...

(b/2) * (sin(THETA2)^2 - sin(THETA1)^2))    % N m

MFf =

179.7115

4	Find the application force from equation 15.14c using the upper sign (-).

Fa = (MFn - MFf) / a    % N

Fa =

1.7430e+003

5	Find the friction torque from equation 15.15.

Tf = MU * w * r^2 * (pmax / sin(THETAm)) * ...

(cos(THETA1) - cos(THETA2))   % N m

Tf =

215.1490

6	The reaction forces are found from equations 15.16.

Rx = w * r * (pmax / sin(THETAm)) * ...

((-1/2) * (sin(THETA2)^2 - sin(THETA1)^2) + ...

MU * ((1/2) * (THETA2 - THETA1) - ...

(1/4) * (sin(2*THETA2) - sin(2*THETA1))))

Rx =

793.9971

Ry = -Fa + w * r * (pmax / sin(THETAm)) * ...

((MU/2) * (sin(THETA2)^2 - sin(THETA1)^2) + ...

((1/2) * (THETA2 - THETA1) - ...

(1/4) * (sin(2*THETA2) - sin(2*THETA1))))

Ry =

4.1336e+003