EXAMPLE 10-4

Selection of Ball Bearings for Combined Radial and Thrust Loads

Problem	Select a deep-groove ball bearing for specified loading and desired life.

Given	Radial load

Fr = 1686;   % lbf

Axial load

Fa = 1012;   % lbf

Design life (million revolutions)

LD = 500;    % million revolutions

Shaft speed

n = 1725;    % rpm

Assumptions	A Conrad-type deep-groove bearing will be used.  The inner race rotates, so the rotation factor in equation 10.20 is V = 1.

V = 1;

Find	A suitable size bearing to give an L10 life of 5E8 revolutions.

Solution	See M-Book Ex10-04.doc and M-File Ex10_04.m.

1	In order to find the required basic load rating we must first find the equivalent radial load.  Unfortunately, this requires that we know what bearing we are going to use in order to calculate the ratio Fa/Co.  This is an iterative process that can be started with an estimate of Fa/Co:

FaoverCo = (5/6)*(Fa/Fr)*LD^-(1/3)

FaoverCo =

0.0630

In Figure 10-24, we see that the nearest tabulated value of Fa/Co to this value is 0.056 so let

FaoverCo = 0.056;

2	From Figure 10-24, find the corresponding value of e

For Fa/Co = 0.056

ee = 0.26;   % (reserving e for powers of 10)

3	Form the ratio Fa/(VFr) and compare to e

Fa / (V*Fr)

ans =

0.6002

which is greater than e

4	Since the ratio in step 3 is > e, extract the X and Y factors from Figure 10-24 and use them to calculate the equivalent load from equation 10.20.

X = 0.56;

Y = 1.71;

P = X*V*Fr + Y*Fa   % lbf

P =

2.6747e+003

5	Calculate the required minimum basic load rating for this application using equation 10.19.

C = P * LD^(1/3)    % lbf

C =

2.1229e+004

6	Go to Figure 10-23 and select a tentative choice for the bearing.  From the figure, choose the 85-mm-bore, #6317, with

C = 21600;   % lbf

Co = 18600;  % lbf

7	Recalculate Fa/Co and determine new values for X and Y.

FaoverCo = Fa/Co

FaoverCo =

0.0544

Interpolating between 0.028 < Fa/Co < 0.056,

X = 0.560;

Y = (FaoverCo - 0.028) / (0.056 - 0.028) * (1.71 - 1.99) + 1.99

Y =

1.7259

8	Recalculate the equivalent load P and the new minimum required basic load rating.

P = X*V*Fr + Y*Fa   % lbf

C = P * LD^(1/3)    % lbf

P =

2.6908e+003

C =

2.1357e+004

9	Since the new minimum required basic load rating C is still less than the basic load rating for the selected bearing, the tentative choice is accepted.  The bearing specifications are:

Catalog No.      Bore          C               Co

6317            85 mm     21600 lb    18600 lb