EXAMPLE 9-8

Determining the Critical Frequencies of a Shaft

Problem	Find the shaft whirl and torsional critical frequencies of the shaft in Example 9-2 and compare them to its forcing frequency.

Given	Refering to Figure 9-5, the shaft and gear are steel, and the sheave is aluminum.  Starting at A, the shaft diameters and lengths are

A to B

d0 = 0.875;  % in

lAB = 1.5;   % in

B to C

d1 = 0.750;  % in

lBC = 3.5;   % in

C to D

d2 = 0.669;  % in

lCD = 1.5;   % in

D to end

d3 = 0.531;  % in

lDE = 1.5;   % in

Other dimensions from Figure 9-5

Distance to gear plane

p = 2.00;    % in

Distance to right support

b = 5.00;    % in

Distance to sheave plane

q = 6.75;    % in

Shaft speed

OMEGArpm = 1725;    % rpm

OMEGA = OMEGArpm * 2* pi / 60   % rad/sec

OMEGA =

180.6416

Gear weight

W1 = 10;     % lbf

Gear moment of inertia

I1 = 0.23;   % lbf in sec2

Sheave weight

W2 = 3;      % lbf

Sheave moment of inertia

I2 = 0.07;   % lbf in sec2

Assumptions	The static deflection of the shaft due to the weights of gear and sheave will be used as an estimate for Rayleigh's method, but the gear and sheave weights will be applied in the directions that give the largest static deflection.  The shaft weight will be ignored.

Solution	See Figures 9-5 and 9-32, M-Book Ex09-08.doc and M-File Ex09_08.m.

1	The deflection of the stepped shaft is found by the same technique used in Example 9-3.  In this instance, the loads are taken as just the weights of the two disks.  But, we will consider the gear weight-force to act downward and the sheave weight-force to act upward since that arrangement better represents the dynamic situation where the inertia forces act outward from the axis in whatever direction increases the deflection.  If we directed both weight forces downward in this case, we would get a smaller maximum deflection and a different curve shape than that of the dynamic deflection.  Figure 9-32 shows the applied weight forces and the deflection curve for this shaft.  The magnitude of the deflection at the gear is

DEL1 = 6e-5;     % in

and at the sheave is

DEL2 = 1.25e-4;  % in

These values are needed in equation 9.25c.

2	Calculate the critical frequency for shaft whirl from equation 9.25c:

g = 32.174 * 12  % in/sec2

OMEGAn = (g*((W1*DEL1 + W2*DEL2)/...

(W1*DEL1^2 + W2*DEL2^2)))^(1/2)    % rad/sec

g =

386.0880

OMEGAn =

2.1312e+003

Note that the magnitudes of the weight forces and their corresponding deflections are all taken as positive regardless of their vector directions in the static deflection case.

3	Compare the critical whirl frequency to the forcing frequency,

OMEGAf = OMEGA

omegaRatio = OMEGAn / OMEGAf

OMEGAf =

180.6416

omegaRatio =

11.7982

This is a very comfortable margin.  If, in this example, the shaft weight is included in both the deflection calculation and in the critical frequency calculation, the critical frequency becomes 20860 rpm, which is 12.1 times the forcing frequency.  Even with the relatively light disks on this shaft, ignoring the shaft weight does not introduce a large error.  Both of these values from Rayleigh's method are higher than the actual natural frequency.

4	To find the torsional critical frequency of a stepped shaft requires that an effective spring constant for the combined stepped sections be found.  The shaft portion of interest is between the sheave and gear (B to C and C to D in Figure 9-5).  The spring constant of any one section is, for

G = 11.5e6;  % psi

kt=G⋅Jl=G⋅π⋅d432⋅l

From B to C

ktBC = (G*pi*d1^4) / (32*lBC)       % lbf in / rad

ktBC =

1.0206e+005

From C to D

ktCD = (G*pi*d2^4) / (32*lCD)       % lbf in / rad

ktCD =

1.5077e+005

Since the shaft sections all have the same torque but different deflections (which sum to the total deflection as shown in equation 9.9b), they act as springs in series, The effective spring constant kteff of the portion of stepped shaft between the two torque loads is found from equation 9.9d:

kteff = ((1/ktBC) + (1/ktCD))^-1    % lbf in / rad

kteff =

6.0863e+004

5	The torsional critical frequency is found from equation 9.28c.

OMEGAn = sqrt(kteff*(I1 + I2)/(I1 * I2))    % rad / sec

OMEGAn =

1.0649e+003

6	Compare the critical torsional frequency to the forcing frequency.

omegaRatio = OMEGAn / OMEGAf

omegaRatio =

5.8953

This is an acceptable margin.