EXAMPLE 4-7

Finding Reactions and Deflection of Statically Indeterminate Beams Using Singularity Functions

Problem	Determine and plot the shear, moment, slope and deflection functions for the beam in Figure 4-22d.  Find maximum deflection.

Given	The load is uniform over part of the beam length.

Beam length

l = 10; % in

Load locations

a = 4;  % in

b = 7;  % in

Distributed load

w = 500;    % lbf / in

Moment of inertia

I = 0.08;  % in4

Modulus of elasticity

E = 30e6;    % psi

Assumptions	Ignore the beam weight as negligible compared to the applied load.

Solution	See Figures 4-22d and 4-27, M-Book Ex04-07.doc and M-File Ex04_07.m

1	Write the equations for the load function in terms of equations 3.17 and integrate the resulting function four times using equations 3.18 to obtain the shear, moment, slope, and deflection functions.

q(x) = R1<x - 0>-1 - w<x - a>0 + R2<x - b>-1 + R3<x - l>-1	(a)

V(x) = R1<x - 0>0 - w<x - a>1 + R2<x - b>0 + R3<x - l>0 + C1	(b)

M(x) = R1<x - 0>1 - w<x - a>2/2 + R2<x - b>1 + R3<x - l>1 + C1x + C2	(c)

?(x) = (R1<x - 0>2/2 - w<x - a>3/6 + R2<x - b>2/2 + R3<x - l>2/2 +

C1x2/2 + C2x + C3 ) / EI	(d)

y(x) = (R1<x - 0>3/6 - w<x - a>4/24 + R2<x - b>3/6  + R3<x - l>3/6 +

C1x3/6 + C2x2/2 + C3x + C4 ) / EI	(e)

2	There are 3 reaction forces and 4 constants of integration to be found.  The constants C1 and C2 are zero because the reaction forces and moments acting on the beam are included in the loading function.  This leaves 5 unknowns to be found.

3	If we consider the conditions at a point infinitesimally to the left of x = 0 (denoted as x = 0-), the shear and moment will both be zero.  The same conditions obtain at a point infinitesimally to the right of x = l (denoted as x = l+).  Also, the deflection y must be zero at all three supports.  These observations provide the 5 boundary conditions needed to evaluate the 3 reaction forces and 2 remaining integration constants: i.e., when x = 0-, V = 0, M = 0; when x = 0, y = 0; when x = b, y = 0; when x = l, y = 0; when x = l+, V = 0, M = 0.

4	Substitute the boundary conditions x = 0, y = 0, x = b, y = 0, and x = l, y = 0 into e.

At x = 0:

y(0) = 0 = R1<0 - 0>3/6 - w<0 - a>4/24 + R2<0 - b>3/6  + R3<0 - l>3/6 +

C3(0) + C4	(f)

C4 = 0;  % lbf in3

At x = b:

y(b) = 0 = R1<b - 0>3/6 - w<b - a>4/24 + R2<b - b>3/6  + R3<b - l>3/6 + C3(b)	(g)

b36⋅R1+b⋅C3=w24⋅l−a4

At x = l:

y(l) = 0 = R1<l - 0>3/6 - w<l - a>4/24 + R2<l - b>3/6  + R3<l - l>3/6  + C3l	(h)

l36⋅R1+l−b36⋅R2+l⋅C3=w24⋅l−a4

5	Two more equations can be written using equations c and b and noting that at point l+, infinitesimally beyond the right end of the beam, both V and M are zero. We can substitute l for l+ since their difference is vanishingly small.

M(l) = 0 = R1<l - 0>1 - w<l - a>2/2 + R2<l - b>1  + R3<l - l>1 	(i)

l⋅R1+l−b⋅R2=w2⋅l−a2

V(l) = 0 = R1<l - 0>0 - w<l - a>1 + R2<l - b>0  + R3<l - l>0  	(j)

R1+R2+R3=w⋅l−a

6	Equations g through j provide 4 equations in the 4 unknowns R1, R2, R3, C3 and can be solved simultaneously.

Coefficient Matrix

Coeff = [b^3/6 0 0 b; l^3/6 (l-b)^3/6 0 l; l l-b 0 0; 1 1 1 0];

Right-Hand-Side Vector

rhs = [(w/24)*(b-a)^4; (w/24)*(l-a)^4; (w/2)*(l-a)^2; w*(l-a)];

Solving…

Soln = Coeff\rhs;

Reaction forces:

R1 = Soln(1)

R2 = Soln(2)

R3 = Soln(3)

R1 =

158.4184

R2 =

2.4719e+003

R3 =

369.6429

Integration constant:

C3 = Soln(4)

C3 =

-1.0527e+003

7	The shear, moment, slope, and deflection diagrams can now be plotted with the same technique used in Examples 3-2B, 3-3B, and 3-4.

Range of x

x = 0:0.005*l:l;

V = R1.*(x>=0).*(x-0).^0 - w.*(x>=a).*(x-a).^1 +...

R2.*(x>=b).*(x-b).^0 + R3.*(x>=l).*(x-l).^0;

M = R1*(x>=0).*(x-0).^1 - (w/2).*(x>=a).*(x-a).^2 +...

R2.*(x>=b).*(x-b).^1 + R3.*(x>=l).*(x-l).^1;

THETA = (1/(E*I)) .* ( (R1/2).*(x>=0).*(x-0).^2 -...

(w/6).*(x>=a).*(x-a).^3 + (R2/2).*(x>=b).*(x-b).^2 +...

(R3/2).*(x>=l).*(x-l).^2 + C3 );

y = (1/(E*I)) .* ( (R1/6).*(x>=0).*(x-0).^3 -...

(w/24).*(x>=a).*(x-a).^4 + (R2/6).*(x>=b).*(x-b).^3 +...

(R3/6).*(x>=l).*(x-l).^3 + C3.*x );

subplot(2,2,1)

plot(x,V)

title('Shear Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Shear, V (lbf)')

subplot(2,2,2)

plot(x,M)

title('Moment Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Moment, M (lbf in)')

subplot(2,2,3)

plot(x,THETA)

title('Slope Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Slope (Rad)')

subplot(2,2,4)

plot(x,y*1000)

title('Deflection Diagram')

xlabel('Distance along beam, x (in)')

ylabel('Deflection (1000th in)')

FIGURE 4-27. Example 4-7 Graphs

8		The maximum deflection will occur at the point in x where the slope of the deflection curve is zero.  From Figure 4-27, we see that this point will be near the middle of the beam where 0 < x < a.  Let the point where θ = 0 be x = c, then, solving for c

θ(c)=0=R12⋅c−02+C3	(l)

Solving for c,

c = sqrt(-2*(C3/R1))  % in

c =

3.6455

9	Use the value of c in equation e to find the largest deflection.

y(c) = ymax = (R1<c - 0>3/6  + C3c ) / EI	(n)

ymax = (1/(E*I))*((R1/6)*c^3 + C3*c)  % in

ymax =

-0.0011