EXAMPLE 11-6

Surface-Stress Analysis of a Spur-Gear Train

Problem	Determine the surface stresses in the gear teeth of the 3-gear train from Examples 11-4 and 11-5.

Given	Transmitted load

Wt = 432;        % lbf

Pinion speed

OMEGAp = 2500;   % rpm

OMEGAp = 2500 * 2*pi;   % rad/min

Train ratio

mG = 3.5;

Tooth numbers

Np = 14;

Ni = 17;

Ng = 49;

Pitch diameters

dp = 2.333;  % in

di = 2.833;  % in

dg = 8.167;  % in

Diametral pitch

pd = 6;      % in-1

Face width

F = 2.000;   % in

Pressure angle

PHIdeg = 25; % deg

PHI = PHIdeg * (2*pi/360)   % rad

PHI =

0.4363

Assumptions	The teeth are standard AGMA full-depth profiles.  The load and source are both uniform in nature.  A gear quality index of Qv = 6 will be used.  All gears are steel with v = 0.28 and E = 30.106 psi.

Qv = 6;

E = 30e6;    % psi

v = 0.28;

Solution	See M-Book Ex11-06.doc and M-File Ex11_06.m.

1	The general formula for tooth-surface stress is equation 11.21:

σb=Cp⋅WtF⋅I⋅d⋅Ca⋅CmCv⋅Cs⋅Cf

Wt, F, Ca, Cm, Cv, and Cs are common to all gears in the set. Cp, d, Cf, and I are potentially different for each pair in mesh.  Use the smaller d of the pair in mesh.

2	Based on the assumption of uniform load and source, the application factor Ka can be set to

Ca = 1;

3	The load distribution factor can be estimated from Table 11-16 based on the assumed face width:

Cm = 1.6;

4	The velocity factor Cv can be calculated from equations 11.16 and 11.17 based on the assumed gear-quality index Qv and the pitch-line velocity Vt.

Pitch-line velocity

Vt = (dp/2) * OMEGAp / 12   % ft / min

Vt =

1.5269e+003

Constants used to calculate Kv

B = 0.25 * (12 - Qv)^(2/3)

A = 50 + 56*(1-B)

B =

0.8255

A =

59.7730

Velocity factor

Cv = (A/(A + sqrt(Vt)))^B

Cv =

0.6602

5	The size factor for all three gears is

Cs = 1;

6	The surface factor for well-finished gears made by conventional methods is

Cf = 1;

7	The elastic coefficient Cp is found from equation 11.23 (same material for both gears in mesh).

Cp = sqrt(E/(2*pi*(1-v^2)))   % psi^(1/2)

Cp =

2.2761e+003

8	The pitting geometry factor I is calculated for a pair of gears in mesh.  Since we have two meshes (pinion/idler and idler/gear) there will be two different values of I to be calculated using equations 11.22.  We will need the pitch diameter and pitch radius of each gear for this calculation.  From the data in Example 11-4:

rp = 0.5 * dp   % in

ri = 0.5 * di   % in

rg = 0.5 * dg   % in

rp =

1.1665

ri =

1.4165

rg =

4.0835

9	For the pinion/idler pair, let Ipi = I, d1 = dp, r1 = rp, and r2 = ri, then for a center distance of

C = rp + ri     % in

C =

2.5830

RHOp = sqrt((rp + 1/pd)^2 - (rp * cos(PHI))^2) - (pi/pd)*cos(PHI)   % in

RHOi = C * sin(PHI) - RHOp  % in

RHOp =

0.3376

RHOi =

0.7540

Ipi = cos(PHI) / (((1/RHOp)+(1/RHOi))*dp)

Ipi =

0.0906

10	For the idler/gear pair, let let Iig = I, d1 = di, r1 = ri, and r2 = rg, then for a center distance of

C = ri + rg     % in

C =

5.5000

RHOi = sqrt((ri + 1/pd)^2 - (ri * cos(PHI))^2) - (pi/pd)*cos(PHI)   % in

RHOg = C * sin(PHI) - RHOi  % in

RHOi =

0.4519

RHOg =

1.8725

Iig = cos(PHI) / (((1/RHOi)+(1/RHOg))*di)

Iig =

0.1165

11	The surface stress for the pinion-idler mesh is then

SIGcp = Cp * sqrt((Wt/(F*Ipi*dp)) * ((Ca*Cm)/Cv) * Cs * Cf)   % psi

SIGcp =

1.1328e+005

12	The surface stress for the idler-gear mesh is then

SIGci = Cp * sqrt((Wt/(F*Iig*di)) * ((Ca*Cm)/Cv) * Cs * Cf)   % psi

SIGci =

9.0665e+004