EXAMPLE 10-1

Sleeve Bearing Design for a Defined Shaft Diameter

Problem	Design sleeve bearings to replace the rolling-element bearings on the shaft shown in Figure 9-5.  The shaft was designed in Example 9-1.

Given	The maximum transverse loads on the shaft at the bearings are 16 lb at R1 and 54 lb at R2.  Since the load at R2 is 4x that at R1, one design can be created for R2 and used also at R1.  The bearings are stationary.

Diameters at R1 and R2

d = 0.591;   % in

Shaft speed

n = 1725;    % rpm

Shaft load

P = 54;      % lbf

Assumptions	Use a clearance ratio of CR = 0.0017 and an l/d ratio of 0.75.  Keep the Ockvirk number at 30 or below, preferably about 20.

CR = 0.0017;

Find	The bearing eccentricity ratio, maximum pressure and its location, minimum film thickness, coefficient of friction, torque, and power lost in the bearing.  Choose a suitable lubricant to operate at 190 F.

Solution	See Figure 9-5, M-Book Ex10-01.doc and M-File Ex10_01.m.

1	Express the speed, given in rpm, in rps and find the tangential velocity U.

n = 1725 / 60           % rev/sec

n = 1725 * 2*pi / 60    % rad/sec

U = (d/2) * n           % in/sec

n =

28.7500

n =

180.6416

U =

53.3796

Note that some equations are unit specific, such as 10.8d and e. Required units are indicated in comments following the Matlab statements.

2	The diametral and radial clearances are found from the given diameter and the assumed clearance ratio:

cd = CR * d     % in

cr = 0.5 * cd   % in

cd =

0.0010

cr =

5.0235e-004

3	The bearing length is found from the assumed l/d ratio of

loverd = 0.75;

l = loverd * d  % in

l =

0.4432

4	Find the experimental eccentricity ratio from equation 10.13b or from Figure 10-10 using the suggested value of ON = 20.

ON = 20;

EPSx = @(ON) 0.21394 + 0.38517*log10(ON) - 0.0008*(ON - 60);

EPSx(20)

ans =

0.7471

5	Find the dimensionless parameter Kε from equation 10.8c.

Keps = @(ON) ON / (4*pi);

Keps(20)

ans =

1.5915

6	The viscosity η of lubricant required to support the design load P can now be found by rearranging equation 10.8b.

ETA = @(ON) (P * cr^2) ./ (Keps(ON) * U * l^3);  % lbf sec / in2

ETA(20)

ans =

1.8419e-006

Enter Figure 10-1 to find that an oil of about ISO VG 100 will provide this value at 190 F.  This oil is equivalent to an SAE 30W engine oil.

7	The average pressure in the oil film is found from equation 10.6d.

pavg = P / (l * d)  % psi

pavg =

206.1378

8	The angle θmax at which the pressure is maximum can be found either from equation 10.7c using the experimental value of εx,

THETAmax = @(ON) acos((1-sqrt(1+24*EPSx(ON).^2))./...

(4*EPSx(ON)));    % rad

THETAmaxDeg = @(ON) acos((1-sqrt(1+24*EPSx(ON).^2))./...

(4*EPSx(ON))) * 360/(2*pi);    % deg

THETAmax(20)

THETAmaxDeg(20)

ans =

2.7790

ans =

159.2273

or it can be read from the experimental curve in Figure 10-12 for ON = 20 as 159 deg.

9	The maximum pressure can be found by substituting θmax in equation 10.7b with z = 0 since it is maximum at the center of the bearing length l.

z = 0   % in

pmax = @(ON) ((ETA(ON)*U)./(0.5*d*cr^2)).*...

((l^2 / 4)-z^2).*((3*EPSx(ON).*sin(THETAmax(ON)))./...

(1+EPSx(ON).*cos(THETAmax(ON))).^3);   % psi

pmax(20)

z =

0

ans =

1.8781e+003

or the ratio of pmax /pavg can be read from the experimental curve in Figure 10-11 for ON = 20 as 9.1 and multiplied by pavg from step 7 above to get the same result.

pmax(20)/pavg

ans =

9.1108

10	Find the angle φ, which locates the θ = 0 to π axis with respect to the applied load P from equation 10.8a.

Phi = @(ON) atan((pi*sqrt(1-EPSx(ON).^2))./(4*EPSx(ON)));  % rad

PhiDeg = @(ON) atan((pi*sqrt(1-EPSx(ON).^2))./(4*EPSx(ON))) * 360/(2*pi);  % deg

Phi(20)

PhiDeg(20)

ans =

0.6100

ans =

34.9488

11	The stationary and rotating torques can now be found from equations 10.9b (substituting 0 for U1) and 10.9a using the angle φ.

Ts = @(ON) ETA(ON).*((d^2*l*U)/cd).*(pi./sqrt(1-EPSx(ON).^2)); % lbf in

Ts(20)

ans =

0.0716

Tr = @(ON) Ts(ON) + P*cr.*EPSx(ON).*sin(Phi(ON));  % lbf in

Tr(20)

ans =

0.0832

12	The power loss in the bearing is found from equation 10.10.

PHI = @(ON) Tr(ON)*n;    % lbf in /sec

PHIhp = @(ON) Tr(ON)*n /6600.4;   % hp

PHI(20)

PHIhp(20)

ans =

15.0310

ans =

0.0023

13	The coefficient of friction in the bearing can be found from the ratio of the shear force to the normal force using equation 10.11.

MU = @(ON) (2*Tr(ON))/(P*d);

MU(20)

ans =

0.0052

14	The minimum film thickness is found from equation 10.4b.  Defining

hmin = @(ON) cr*(1-EPSx(ON));   % in

hmin(20)

ans =

1.2707e-004

This is a reasonable value since the composite rms surface roughness (see equation 10.14a in the next section) needs to be no more than about a third to a fourth of the minimum film thickness to avoid asperity contact (see Figure 10-13) and a 30-40 µin Rq finish or better is easily obtainable by precision milling, grinding, or honing.

15	Most of the equations above were written as functions of the Ocvirk number ON.  This was done so that, at this point, we could see how they vary with changes in ON.  Let ON vary over the range 10 to 75, then we have

ON = 10:5:75;

i = 1:length(ON);

ONcol(i,1)=ON(i);

EPSxCol(i,1) = EPSx(ON);

ETAcol(i,1) = ETA(ON) * 1e6;     % microreyn

pmaxCol(i,1) = pmax(ON) * 1e-3;  % ksi

PHIcol(i,1) = PHIhp(ON) * 1e3;   % milli hp

hminCol(i,1) = hmin(ON) * 1e6;   % micro in

ALL = [ONcol EPSxCol ETAcol pmaxCol PHIcol hminCol]

ALL =

10.0000    0.6391    3.6838    1.4129    3.7133  181.2931

15.0000    0.7029    2.4559    1.6183    2.7666  149.2306

20.0000    0.7471    1.8419    1.8781    2.2773  127.0656

25.0000    0.7804    1.4735    2.1923    1.9752  110.3238

30.0000    0.8069    1.2279    2.5674    1.7687   97.0124

35.0000    0.8287    1.0525    3.0130    1.6182   86.0683

40.0000    0.8470    0.9209    3.5418    1.5033   76.8568

45.0000    0.8627    0.8186    4.1697    1.4127   68.9687

50.0000    0.8763    0.7368    4.9160    1.3396   62.1245

55.0000    0.8883    0.6698    5.8044    1.2794   56.1248

60.0000    0.8988    0.6140    6.8638    1.2293   50.8225

65.0000    0.9082    0.5667    8.1295    1.1872   46.1058

70.0000    0.9166    0.5263    9.6442    1.1515   41.8877

75.0000    0.9242    0.4912   11.4602    1.1212   38.0996

16	A safety factor against asperity contact can be estimated by using the data in the table above.  Choosing a minimum film thickness of, say, 42 µin, the required Ocvirk number, eccentricity ratio and load P to reduce the film thickness to that value for the same viscosity found for ON = 20 are

From the matrix above for ON = 70

hmin = 42   % micro in

EPSx(70)

hmin =

42

ans =

0.9166

and at ON = 20

eta = ETA(20) % reyn (= lbf sec / in2)

eta =

1.8419e-006

then from equation 10.8b

P70 = eta * Keps(70)*U*l^3 / cr^2   % lbf

P70 =

189

The resulting safety factor is,

N = P70/P

N =

3.5000

which is an acceptable reserve for overloads.

17	If this safety factor had indicated that a small overload could put the bearing in trouble, redesigning the bearing for a lower Ocvirk number would give more margin against failure under overloads.  Equation 10.11c, repeated here, shows what could be changed to reduce ON:

ON=pavgη⋅n⋅dl2⋅cdd2

It would require some combination of: decreasing the clearance ratio, decreasing the d/l ratio, or using a higher viscosity oil.  Assuming the rotational speed, load, and shaft diameter remain unchanged, the bearing length could be increased or the diametral clearance reduced as well as η increased to improve the design.