# H.1	INTRODUCTION TO MATLAB

Matlab is a very commonly used program that has applicability to a wide range of problems. One of the key features of Matlab is the interactive environment provided by the Command Window as shown in Figure H.1. The Command Window is so intuitive that it allows you to quickly start solving problems with little instruction. On the other hand, Matlab is an extremely powerful program that can handle very complex problems. It would take considerable study to fully understand all that Matlab is capable of doing.

Figure H.1. The Matlab Command Window.

Matlab is a procedural program, that is, it performs calculations in the order that the equations are entered, one after another. The equations can be entered in the Command Window and solved immediately, or a list of equations can be created in a text editor and saved as a script, called a script m-file. Matlab provides a text editor, although any editor that can create a simple text file can be used, just save the file with a ".m" extension. When a script m-file is executed, the equations are solved in order they are listed in the file, from beginning to end.

Another type of m-file is a function m-file. Function m-files allow you to write your own functions, to supplement the extensive library of functions that come built-in with Matlab.

Matlab doesn't handle units, so Matlab users have to be careful to watch the units in their calculations and provide conversion factors as needed. In the examples included with this text, the units of the result are included as comments after each equation. For example, the following Matlab statement defines the gravitational acceleration in SI units:

>> g = 9.8     % m/s2

The ">>" is the command prompt in the Command Window that indicates that Matlab is ready for you to enter an equation, and the percent symbol "%" is a comment character that indicates that the characters to the right of the percent symbol should not be evaluated as part of the equation.

When you enter an equation at the prompt in the Command Window and then press [Enter], Matlab immediately evaluates the equation and displays the result. Then it displays another prompt for another equation.

>> g = 9.8   % m/s2

g =

9.8000

>>

Matlab displays the variable name and the result on separate lines. When variables are assigned values, such as the variable g here, they retain their values and may be reused later in other equations. Variables declared in this way are called session variables are available throughout your entire Matlab session. You may want to clear your session variables between problems to ensure that you do not accidentally carry information from one problem to the next. To do so, enter clear at the command prompt, or use menu options Edit / Clear Workspace.

The best way to learn Matlab is to start using it. The following examples are designed to illustrate some of the most commonly used features. While each of these examples is available on the disk that accompanies this text, you will learn Matlab faster by trying these examples yourself.

Figure H.2. Stainless Steel Cylindrical Container (Cooking Pot)

## Solving Simple Models

We wish to design a 1-liter-capacity cooking pot, as shown in Figure H.2, to be made of 1-mm-thick stainless steel. Ultimately we would like to minimize the amount of stainless steel used. We will start with the simplest possible model.

EXAMPLE H-1

Simple Equation-Solving Using Matlab – Part A

Problem	Find the height and empty weight of an open-top, cylindrical container (a cooking pot) for a desired volume.

Given	Volume		1 liter = 1000 cm3

Inside diameter	12 cm

Wall thickness	0.1 cm

Assumptions	The material is stainless steel, mass density = 7.75 g/cm3.

Solution	See Figure F-1, Tables F-1 and F-2, and Matlab M-File Example1.m

1	The contained column is the base area times the height. The base area can be calculated from the known inside diameter.

>> d = 12;   % cm

>> basearea = (pi/4) * d^2   % cm2

basearea =

113.0973

Matlab responds by displaying the result, that the base area is 113.0973 cm2. Notice that pi is a predefined constant in Matlab. Remember that Matlab does not handle units; the units shown in the equations are comments (following the % symbol) and are not interpreted as part of the equation. The semicolon ; following the d = 12 equation was used to tell Matlab to evaluate the equation, but not to report the result.

2	The height can then be determined from the base area and the volume.

>> vol = 1000;   % cm3

>> H = vol / basearea   % cm

H =

8.8419

3	Since we want to know the weight of the cooking pot, we need to know the surface area.

>> surface = basearea + pi*d*H   % cm2

surface =

446.4307

4	The metal volume can be calculated as surface area times metal thickness.

>> thick = 0.1;   % cm

>> metalVol = surface * thick   % cm3

metalVol =

44.6431

5	The mass of the metal is then determined by multiplying the metal volume by the density.

>> dens = 7.75;   % g/cm3

>> weight = metalVol * dens   % g

weight =

345.9838

The cooking pot is found to weigh 346 grams.

In Example H-1 the equations were solved in the Command Window. An alternative is to use the built-in text editor to create a script m-file of all of the equations needed to solve the problem. This is useful if you will need to solve the same (or very similar) problem again.

As a script m-file, this problem might look like the lines shown in the Matlab editor in Figure H.3.

Figure H.3. A script in the Matlab editor (file: Example1.m).

You can run the script to cause each equation in the script m-file to be evaluated by pressing the run button indicated in Figure H.3. Alternatively, use can use menu options Debug / Run, or the keyboard shortcut [F5]. The results are displayed in the Command Window. You must save the file with a .m extension before running the script.

## Modifying Script m-Files to Solve Related Problems

One of the advantages of using a script file is that they can be edited to quickly solve similar problems. In the next example, the script is modified to solve for diameter instead of height.

EXAMPLE H-2

Simple Equation-Solving Using Matlab – Part B

Problem	Find the height and empty weight of an open-top, cylindrical container (a cooking pot) for a desired volume.

Given	Volume		1 liter = 1000 cm3

Inside height	10 cm

Wall thickness	0.1 cm

Assumptions	The material is stainless steel, mass density = 7.75 g/cm3.

Solution	See Figure F-1, Tables F-1 and F-2, and Matlab M-File Example2.m

1	The only difference between this example and the previous one is that we have specified the pot's height and will calculate the diameter needed for a volume of 1 liter.

2	The script m-file created in Example 1 is quickly modified to solve for d rather than H. The modifications are highlighted in the following listing.

% Determine the Diameter and Weight of an Open Cylinder of Known Volume

% File: Example2.m

%

% Given

vol = 1000;   % cm3

H = 10;       % cm

thick = 0.1;  % cm

%

% Assumptions

dens = 7.75;  % g / cm3

%

% 1

basearea = vol / H            % cm

% 2

d = sqrt((4/pi) * basearea)   % cm

% 3

surface = basearea + pi*d*H   % cm2

% 4

metalVol = surface * thick    % cm3

% 5

weight = metalVol * dens      % g

3	Running the script produces the following results in the Command Window.

basearea =

100

d =

11.2838

surface =

454.4908

metalVol =

45.4491

weight =

352.2303

## Using Iteration and Root Finding

We now complicate the problem by introducing a height-to-diameter ratio constraint (H/D). The diameter required to generate the correct volume and maintain the desired height-to-diameter ratio can be determined using an iterative solution. Matlab provides a built-in function called fzero() to find the value that makes a function evaluate to zero. This is Matlab's iterative solution function.

EXAMPLE H-3

Iterative Equation-Solving Using Matlab's fzero() Function

Problem	Find the diameter, height and weight of an open-top, cylindrical container (a cooking pot) for a desired volume and height/diameter ratio (H/D).

Given	Volume		1 liter = 1000 cm3

H/D			0.6

Wall thickness	0.1 cm

Assumptions	The material is stainless steel, mass density = 7.75 g/cm3.

Solution	See Figure F-1, Tables F-1 and F-2, and Matlab M-File Example3.m

1	Matlab's fzero() function searches for the value that causes a function to evaluate to zero. To use this function, we need to combine the volume function

vol(D,H)=π4D2H

and the height-to-diameter constraint

HD=0.6

to create a volume function that depends on only one variable.

vol(D)=π4D2⋅0.6 D

2	But the desired volume is known, vol = 1 liter = 1000 cm3.

vol=π4D2⋅0.6 D

This equation could be solved directly for D, but it can also be solved using an iterative approach. In order to use fzero(), the equation must be rearranged such that the desired value of D causes the function to evaluate to zero.

0=vol−π4D2⋅0.6 D

Written as a function, call it volFUNC, we get

volFUNC(D)=vol−π4D2⋅0.6 D

3	The next step is to begin solving the problem with Matlab. We start by entering the known values.

>> vol = 1000;       % cm3

>> HDratio = 0.6

>> thick = 0.1       % cm

>> dens = 7.75;      % g / cm3

4	Now, enter the function to be evaluated.

>> volFUNC = @(D) vol-((pi/4) * (D^2 * (0.6 * D)));   % D [=] cm

This statement could be read something like this…

Create a function called volFUNC that is  =  a function of D   @(D)  and that function is defined as

vol-((pi/4) * (D^2 * (0.6 * D)))

Do not display the result of this function assignment  ; and indicate (using a comment  %  ) that the variable D has units of cm  D [=] cm  .

5	Like all iterative methods, the fzero() function requires an initial value, or guess, for the iteration variable.

>> Dguess = 10;   % cm

6	Next, we use the fzero() function to find the value of D.

>> D = fzero(volFUNC,Dguess)   % cm

D =

12.8505

A diameter of 12.9 cm provides a volume of 1000 cm3 with an H/D ratio of 0.6.

7	Now, use the H/D ratio to find the corresponding height.

>> H = D * HDratio   % cm

H =

7.7103

8	Next, find the surface area of the pot.

>> basearea = vol / H   % cm2

basearea =

129.6968

>> surface = basearea + pi*d*H   % cm2

surface =

420.3680

9	The volume and weight of the metal can now be calculated.

>> metalVol = surface * thick   % cm3

metalVol =

42.0368

>> weight = metalVol * dens   % g

weight =

325.7852

As a script m-file, the previous example would look like this:

% Iterative Equation-Solving Using Matlab's fzero() Function

% File: Example3.m

%

% Given

vol = 1000;      % cm3

HDratio = 0.6;   % cm

thick = 0.1;     % cm

%

% Assumptions

dens = 7.75;     % g / cm3

%

% 4

volFUNC = @(D) vol-((pi/4) * (D^2 * (0.6 * D)));   % d [=] cm

% 5

Dguess = 10                    % cm

% 6

D = fzero(volFUNC(D),Dguess)   % cm

% 7

H = D * HDratio                % cm

% 8

basearea = vol / H             % cm2

surface = basearea + pi*d*H    % cm2

% 9

metalVol = surface * thick     % cm3

weight = metalVol * dens       % g

The fzero() function tries to find the value of the function variable that causes the function's return value to be zero. It does this by finding values if the function variable (D in Example H-3) that return positive and negative results. The function then works towards the middle of the interval to find the value that makes the function zero. Because of this, the function must find two values of the function variable that return results with different signs. If you provide a single starting value, such as Dguess in Example H-3, the fzero() function will try to find another value that causes the function to return a result with the opposite sign. If fzero() cannot produce a result with the opposite sign, an error message is returned.

You can also provide fzero() with exact limits of the range you want the function to search. For example, if we believe the value of D that makes the volFUNC function return zero is between 10 and 15 cm, we could instruct the fzero() function to search that range by providing the range [10 15] in place of Dguess.

D = fzero(volFUNC(D),[10 15])   % cm

Whenever you supply a range, one of the limits must evaluate to a positive result, and the other to a negative result. That is, the range must include the solution!

### Other ways to define the function evaluated by fzero()

- Anonymous function

It is also possible to put the function definition directly in the fzero() statement, eliminating the definition of volFUNC.

D = fzero(@(D) vol-((pi/4) * (D^2 * (0.6 * D))),[10 15])   % cm

Functions used in this manner have no names, and are called anonymous functions in Matlab.

- Function m-file

The function definition could also be written as a function m-file. A function m-file is a file that contains a user-written function. A function m-file always starts with the word function, and should be saved using the function name as the file name, with a .m file extension. For example, we could create a volFILE.m file containing the function to be evaluated by fzero().

User-Written Function Listing: volFILE.m

function ret = volFILE(D)

vol = 1000;      % cm3

ret = vol-((pi/4) * (D^2 * (0.6 * D)));   % D [=] cm

The ret variable is the function's return value. (Any variable name can be used.) The fzero() function is instructed to use volFILE.m using either of the following commands:

D = fzero(@volFILE,Dguess)    % cm

D = fzero(@volFILE,[10 15])   % cm

As a final modification, it is desirable to pass the desired volume vol into the m-file rather than setting the value in the m-file. The m-file is modified to include vol as an argument to the function as shownin the listing below.

User-Written Function Listing: volFILE.m (modified)

function ret = volFILE(D, vol)

ret = vol-((pi/4) * (D^2 * (0.6 * D)));   % D [=] cm

Now, the volume is passed into the volFILE() function as an argument. But fzero() assumes the function it calls will have only one argument. To get the second argument passed into volFILE() you need to create an anonymous function in fzero() that calls volFILE().

D = fzero(@(D) volFILE(D,vol),[10 15])   % cm

The characters @(D) volFILE(D,vol) translate approximately like this:

Fzero() calls "a function of D"   @(D)   , and that function looks like this:  volFILE(D,vol) .

In this manner fzero() calls the anonymous function @(D) which is a function of D only, as required. But that anonymous function calls volFILE(D, vol), and includes the diameter and volume as arguments. This way fzero() doesn't know anything about vol, but vol still gets passed to volFILE(). You need to be sure that vol is assigned a value before using the fzero() function.

vol = 1000;   % cm

D = fzero(@(D) volFILE(D,vol),[10 15])   % cm

## Vectors and Plotting in Matlab

The next example illustrates how a function can be evaluated for a range of values to create a data set for plotting. This approach can be very useful for visualizing the values that a function returns, and is a simple way to find a maximum or minimum value. In the following example, we will use a plot to find the height-to-diameter ratio that minimizes the weight of the cooking pot.

Example H-4

Vectors and Plotting in Matlab

Problem	Find the height/diameter ratio and dimensions of an open-top, cylindrical container that will minimize its weight for a given volume. Plot the variation of weight with the height/diameter ratio.

Given	Volume		1 liter

Wall thickness	0.1 cm

Assumptions	The material is stainless steel with a mass density 7.75 g/cm3. The ratio H/D will be varied from 0.1 to 2.0 in increments of 0.1.

Solution	See Figures F-1 and F-2, and the Matlab M-Files Example4a.m and Example4.m.

0	Before we start the solution, the given and assumed values are defined.

>> vol = 1000;      % cm3

>> thick = 0.1;     % cm

>> dens = 7.75;     % g / cm3

1	Ultimately, we want to use a range of ratio values to calculate a range of weights, and plot them to find the ratio value that minimizes the weight. But first, let's work through the required calculations using a single value of ratio.

>> ratio = 0.1;

2	The relationship between volume, diameter and height is

vol=π4D2H

The H/D ratio can be used to eliminate H from the volume equation.

ratio=HDvol=π4D2ratio⋅Dvol=π⋅ratio4D3

This equation can be solved directly for D.

D=4⋅volπ⋅ratio13

Or, as a Matlab statement,

>> D = ((vol * 4) / (pi * ratio))^(1/3)   % cm

D =

23.3509

3	Then, the H/D ratio can be used to determine H.

>> H = D * ratio   % cm

H =

2.3351

4	Once H and D are known, the metal volume can be calculated.

>> metalVol = (((pi/4) * D^2) + (pi * D * H)) * thick   % cm3

metalVol =

59.9549

5	The mass density can be used to determine the weight from the metal volume.

>> weight = metalVol * dens   % g

weight =

464.6505

6	At this point we have a working solution for a single value of ratio. To solve the problem for a series of ratio values, we create a vector of ratio values.

>> ratio = 0.1:0.1:2.0

This creates a twenty-element vector called ratio. The Matlab statement used to create the ratio vector includes a start_value (0.1), a step_value (0.1), and a stop_value (2.0) all separated by colons.

ratio =

Columns 1 through 10

0.10  0.20  0.30  0.40  0.50  0.60  0.70  0.80  0.90  1.00

Columns 11 through 20

1.10  1.20  1.30  1.40  1.50  1.60  1.70  1.80  1.90  2.00

7	If you send a vector of ratio values into the D equation, Matlab will attempt to use matrix math operations on the vector to evaluate D. However, you cannot raise a vector to the 1/3 power, so sending a vector into the D equation will generate an error. The D equation has to be modified to tell Matlab to use element-by-element math operations, not matrix operations. (Matlab calls this array math as opposed to matrix math.) You tell Matlab to use element-by-element math by placing a decimal point before an operator, such as the .*  ./  and .^ operators.

>> D = ((vol * 4) ./ (pi .* ratio)).^(1/3)   % cm

D =

Columns 1 through 10

23.35 18.53 16.19 14.71 13.65 12.85 12.20 11.67 11.22 10.83

Columns 11 through 20

10.49 10.19  9.93  9.68  9.46  9.26  9.08  8.91  8.75  8.60

8	The other equations involving ratio, or D, or any vectors created using ratio or D must be similarly modified to instruct Matlab to use element-by-element math operations.

>> H = D .* ratio;   % cm

>> metalVol = (((pi/4) .* D.^2) + (pi .* D .* H)) * thick;   % cm3

>> weight = metalVol .* dens;   % g

9	Finally, we can plot the metal weight as a function of ratio.

>> plot(ratio,weight)

>> xlabel('H/D ratio')

>> ylabel('weight (g)')

>> ylim([300 400])

In these statements, the plot function creates the basic plot, which Matlab displays in a separate window. The xlabel() and ylabel() functions add the axis labels to the plot, and the ylim([300 400]) statement restricts the range of displayed y values to the range 300 to 400 grams. The result is shown in Figure H-2.

Figure H-2. Variation of Weight with Height-to-Diameter Ratio in Example H-4.

In this example, the process for calculating weight was performed for a single value of ratio in steps 1 through 5 to help make the changes required to work with a vector of ratio values more obvious. It is possible to solve this problem using the ratio vector from the beginning. That approach is summarized in the script m-file listed below. Notice the semicolons after all of the statements before the plot() function is called. This inhibits the output of all of the intermediate vector results, which can take up a lot of space.

The statements used to create Figure H.2 are presented as a script m-file, Example4.m.

% Vectors and Plotting in Matlab

% File: Example4.m

%

% Given

vol = 1000;      % cm3

thick = 0.1;     % cm

%

% Assumptions

dens = 7.75;     % g / cm3

%

% 1

ratio = 0.1:0.1:2.0;

% 2

D = ((vol * 4) ./ (pi .* ratio)).^(1/3);   % cm

% 3

H = D .* ratio;   % cm

% 4

metalVol = (((pi/4) .* D.^2) + (pi .* D .* H)) * thick;   % cm3

% 5

weight = metalVol .* dens;   % g

% 6

plot(ratio,weight)

xlabel('H/D ratio')

ylabel('weight (g)')

ylim([300 400])

%

### Using Built-In Functions

We now modify the script m-file from the preceding example to add the ability to extract the optimum solution values from the solution vectors (ratio, D, H, weight) without the need for a plot. We will also decrease the step size used to create the ratio vector to create a lot more data points over the same range to help obtain a more precise result.

Example H-5

Vectors and Built-In Functions in Matlab

Problem	Find the height/diameter ratio and dimensions of an open-top, cylindrical container that will minimize its weight for a given volume. Extract the optimum values from the solution vectors.

Given	Volume		1 liter

Wall thickness	0.1 cm

Assumptions	The material is stainless steel with a mass density 7.75 g/cm3. The ratio H/D will be varied from 0.1 to 2.0 in increments of 0.1.

Solution	See Matlab M-File Example5.m.

0	Before we start the solution, the given and assumed values are defined.

>> vol = 1000;      % cm3

>> thick = 0.1;     % cm

>> dens = 7.75;     % g / cm3

1	We create a vector of ratio values over the range 0.1 to 2.0 with a step size of 0.025.

>> ratio = 0.1:0.025:2.0;

2	We use the vector of ratio values to solve for a vector of weight values (as in the preceding example).

>> D = ((vol * 4) ./ (pi .* ratio)).^(1/3);                  % cm

>> H = D .* ratio;                                           % cm

>> metalVol = (((pi/4) .* D.^2) + (pi .* D .* H)) * thick;   % cm3

>> weight = metalVol .* dens;                                % g

3	We now use Matlab's min() function to find the minimum value in the weight vector.

>> minwt = min(weight)          % g

minwt =

340.5176

4	In order to determine which element of the weight vector contains the minimum value, we again use Matlab's min() function, but we request both the minimum value and the array index of the minimum value.

>> [minwt i] = min(weight)      % [g  index]

minwt =

340.5176

i =

17

5	Now that we know that the minimum weight is element 17 of the weight vector, we can find the corresponding elements of the ratio, D, and H vectors. We will call these the optimal results.

>> optRatio = ratio(i)

optRatio =

0.5000

>> optD = D(i)      % cm

optD =

13.6557

>> optH = H(i)      % cm

optH =

6.8278

## User-Defined Functions

There are two types of user-defined functions that can be used in Matlab, function m-files and anonymous functions. Function m-files are more common and will be discussed first.

### Function M-Files

Function m-files are user-written functions that are stored in their own files, called m-files because they use the .m file extension. Both script m-files and function m-files are stored with the .m extension, but you can tell them apart because the first word of a function m-file is the keyword function. Once a function m-file has been written and saved, it can be used just like any of Matlab's built-in functions.

Note: User-written m-files are typically stored in the "Work" subdirectory of the Matlab folder. However on a shared system, you may need to keep your m-files in another location. Use the Set Path option from the Files menu to let Matlab know where to find your m-files.

The function in a function m-file has a name, and the function name and file name should be identical, except that the file name includes the .m extension. A function m-file can accept input arguments and return output values. The first line of a function m-file includes the following items, in the following order:

- the keyword "function"

- the names of the output variable(s) (in [ ] if more than one value is returned)

- the function name (same as the file name)

- the input argument(s)

As an example of a function m-file, we will create a function that creates a vector of evenly-spaced values between a minimum value min and a maximum value max with a step size step. This is what was done in the last two examples to create the ratio vector. Here we will use a range with a minimum value of 1.0, a maximum value of 2.0, and a step size of 0.25. There is really no good reason to write this function because you can accomplish the same result much more simply using the Matlab command

>> ratio = 1.0:0.25:2.0;

except, this simple function will demonstrate the following:

- the structure of a function m-file

- the use of a for loop to iterate within a function.  Note: for loops are used to repeat a set of calculations a set number of times.

- how a function is called from the Matlab Command Window

Example H-6

Function M-Files in Matlab

Problem	Write a function (i.e., a function m-file) that receives a minumum value, maximum value and step size as input arguments, then calculates the elements of a vector of uniformly-spaced values in the specified range, and returns the vector as an output value.

Solution	See Matlab function M-Files: createVector.m and createVector2.m.

1	First, start a new m-file by using menu options File / New / M-File. This will open the Matlab editor with the new (empty) file available for editing. (The file has not yet been saved.)

2	We start writing the function by entering the first line of the m-file, which contains the keyword "function", the name of the output variable, the function name, and the input arguments.

function ret = createVector(min, step, max)

3	First, create a loop that runs from min to max with a step size of step.

for j = min:step:max

end

Notice the syntax of the for loop. The loop starts with the keyword for and ends with the keyword end. A new variable is used to keep track of progress of the loop. You can use any variable name you like, but j was used in this example. The values of j range from min to max with a step size of step. This is indicated by the min:step:max range designation. Any calculations you want to have performed repeatedly go between the for and end statements.

4	The only "calculation" we want to do is to assign the value of j to the ret vector each time through the loop. But to do this, we need to know the current array element. So, we add a counting variable, i, to keep track of the current array element.

i = 1;   % initial value of the array index variable

for j = min:step:max

i = i + 1;

end

Now, every time the for loop iterates, i is incremented by one.

5	The last step is to assign the value of j to the ith element of ret. This is done before i is incremented so that the first time through the loop, the first value of j ( which is equal to min) will be assign to ret(1), the first element of the ret vector.

i = 1;   % initial value of the array index variable

for j = min:step:max

ret(i) = j;

i = i + 1;

end

The complete function listing is this:

function ret = createVector(min, step, max)

i = 1;   % initial value of the array index variable

for j = min:step:max

ret(i) = j;

i = i + 1;

end

Notice the semicolons at the end of the assignment lines. These prevent the results of these lines from being displayed when the function is used. It is common, after a function has been debugged, not to show output from calculations performed inside functions.

You need to save the function as createVector.m. If you select Save As from the File menu, Matlab will automatically suggest that you use that name by filling in the name field on the Save As dialog.

6	We can now use the function just like any other Matlab function.

>> ratio = createVector(1,0.25,2)

ratio =

1.0000    1.2500    1.5000    1.7500    2.0000

Or, the arguments sent to the createVector function can be declared in advance and passed in as variables.

>> min = 1;

>> max = 2;

>> step = 0.25;

>> ratio = createVector(min,step,max)

ratio =

1.0000    1.2500    1.5000    1.7500    2.0000

There are other ways to write this function. For example, instead of the for loop, we could use a while loop and keep iterating until the next value of ret would be greater than max. We will call this version createVector2().

function ret = createVector2(min, step, max)

ret(1) = min;   % set the first element of ret vector

i = 1;          % initialize the counting variable

while (ret(i) + step) <= max

ret(i+1) = ret(i) + step;

i = i + 1;

end

Both versions produce the same results.

### Working with Arrays as Lists

Automatically choosing data from a table can be handy (after getting the data into the computer). Two methods for selecting data from a stored table will be presented in the following examples. The data in the tables will include a few property values for three metals (from Table C-1).

The first method of selecting data (Example H-7A) uses two arrays to hold the table; one for the metal names, and one for the property values. Two arrays are required because Matlab does not like to mix text strings with numbers in arrays. This does not pose a problem for selecting the data because, once the correct row has been identified in the name array, the same row holds the values in the numeric array. However maintaining two arrays can be problematic because, as new rows are added, it is possible to get the names and data values out of sync. The second method avoids this problem by keeping the names connected with the data.

The second method for selecting data uses a structure (a collection of related values) to holds all of the data for each metal – including the metal name. It is possible to search for the desired metal name and return the data in that metal's structure. This will be demonstrated in Example H-7B.

Example H-7A

Selecting Data from Arrays in Matlab – Method 1

Problem	Write a function (i.e., a function m-file) to return the material properties of a selected material from an array.

Given	Material properties from Table C-1

Property	Aluminum	Steel	Copper

Density	2.8	7.8	8.9	g/cm3

Modulus of Elasticity	71.7	206.8	120.7	GPa

Modulus of Rigidity	26.8	80.8	44.7	GPa

Solution	See Matlab M-File Example7A.m.

1	First, create the array that holds the metal names.

>> MetalNames = cellstr(char('Alum','Steel','Copper'))

MetalNames =

'Alum'

'Steel'

'Copper'

The MetalNames array is a special type of array, called a cell array, that is used for text strings (and other non-numeric data). Each text string (i.e., each name) in a cell array must have the same number of characters. The char() function adds spaces to the end of short strings to ensure that all strings are the same length. Then the cellstr() function is used to create a cell array from a list of text strings.

2	Next, create the array to hold the metal property values. The semicolons separate rows in the matrix, and each metal's data is on a separate row.

>> MetalProps = [2.8 71.7 26.8; 7.8 206.8 80.8; 8.9 120.7 44.7]

MetalProps =

2.8000   71.7000   26.8000

7.8000  206.8000   80.8000

8.9000  120.7000   44.7000

3	Now we need a way to find the data for the right metal. That is, we need to identify the correct row. Matlab does not provide a direct way to do this, but it can easily be accomplished with a simple function m-file. This function has been called findName().

function row = findName(Name,Table)

row = 0;

n = length(Name);

[r c] = size(Table);

for i = 1:r

if strncmpi(Table(i),Name,n) == 1

row = i;

return

end

end

The findName() function receives two input arguments: Name and Table. Name is a test string such as 'Alum' or 'Steel'. The Table will hold the cell array of metal names. The function tries to find the row in the Table that contains the desired Name, and returns the row number as the function's output variable.

- The row variable is initially set to zero so that, if no match if found, the function will return a value of zero.

- The length of the Name string is determined for the string comparison that will be performed later.

- The size() function is used to find the number of rows in the Table of property values.

- The for loop is set up to step through all of the rows in the Table.

- The contents of each row of the Table are compared to the Name string. If a match is found, then the function's return variable row is set to the current value of the loop index i, and the return statement sends program flow out of the loop and out of the findName() function.

- If no match is found in the Table, then the for loop ends without ever resetting the value of row, and the function returns row = 0.

4	The findName() function can now be used in the Command Window to find the row that contains the desired data. Here, we search for the name 'Steel'.

>> row = findName('Steel',MetalNames)

row =

2

5	All that remains is to report the property values for steel.

>> Name = MetalNames(row)

Name =

'Steel'

>> Dens = MetalProps(row,1)   % g / cm3

Dens =

7.8000

>> ModE = MetalProps(row,2)   % GPa

ModE =

206.8000

>> ModR = MetalProps(row,3)   % GPa

ModR =

80.8000

In script form, this example might look like this:

% Selecting Data from Arrays in Matlab – Method 1

% File: Example7A.m

% 1 – construct the names array

MetalNames = cellstr(char('Alum','Steel','Copper'));

% 2 – construct the data array

% Property values from Table C-1

% col 1: Density (g/cm3)

% col 2: Modulus of Elasticity (GPa)

% col 3: Modulus of Rigidity (GPa)

MetalProps = [2.8 71.7 26.8; 7.8 206.8 80.8; 8.9 120.7 44.7];

%3 - create the findName() function m-file

%4 – find the desired row

row = findName('Steel',MetalNames);

%5 – output the results

Name = MetalNames(row)

Dens = MetalProps(row,1)   % g / cm3

ModE = MetalProps(row,2)   % GPa

ModR = MetalProps(row,3)   % GPa

The second method for locating tabulated information uses structures. Structures are collections of related information which can be identified by their structure number, and individual elements in a structure by the structure number and the field name. This is illustrated in Example H-7B.

Example H-7B

Selecting Data from Structures in Matlab – Method 2

Problem	Write a function (i.e., a function m-file) to return the material properties of a selected material from a data structure array (i.e., array of data structures).

Given	Material properties from Table C-1

Property	Aluminum	Steel	Copper

Density	2.8	7.8	8.9	g/cm3

Modulus of Elasticity	71.7	206.8	120.7	GPa

Modulus of Rigidity	26.8	80.8	44.7	GPa

Solution	See Matlab M-File Example7B.m.

1	First, delete any MetalProps that might already have been defined.

>> clear MetalProps;

2	Next, create the data structures.

>> MetalProps.Name = 'Alum';

>> MetalProps.Dens = 2.8;      % g/cm3

>> MetalProps.ModE = 71.7;     % GPa

>> MetalProps.ModR = 26.8;     % GPa

>> MetalProps(2).Name = 'Steel';

>> MetalProps(2).Dens = 7.8;      % g/cm3

>> MetalProps(2).ModE = 206.8;    % GPa

>> MetalProps(2).ModR = 80.8;     % GPa

>> MetalProps(3).Name = 'Copper';

>> MetalProps(3).Dens = 8.9;      % g/cm3

>> MetalProps(3).ModE = 120.7;    % GPa

>> MetalProps(3).ModR = 44.7;     % GPa

The syntax is StructureName(structureNumber).fieldName, except for the first structure (structure 1 is assumed). So the structure array name is MetalProps. Structure MetalProps(1) will hold aluminum data, MetalProps(2) will hold steel data, and MetalProps(3) will hold copper data. Each structure has four fields: Name, Dens, ModE, and ModR.

3	Now we need a way to find the right structure for the desired metal. Again, we can write a simple function m-file to do this task. This function has been called findName2().

function structNum = findName2(Name,StructureArray)

structNum = 0;

n = length(Name);

[c s] = size(StructureArray);

for i = 1:s

if strncmpi(StructureArray(i).Name,Name,n) == 1

structNum = i;

return

end

end

The findName2() function receives two input arguments: Name and StructureArray. Name is a test string such as 'Alum' or 'Steel'. The StructureArray holds the metal names and properties. The function tries to find the number of the structure in StructureArray that contains the desired Name, and returns the structNum number as the function's output variable.

- The structNum variable is initially set to zero so that, if no match if found, the function will return a value of zero.

- The length of the Name string is determined for the string comparison that will be performed later.

- The size() function is used to find the number of data structures in StructureArray.

- The for loop is set up to step through all of the structures in StructureArray.

- The contents of the .Name field of each structure of the StructureArray are compared to the Name string. If a match is found, then the function's return variable structNum is set to the current value of the loop index i, and the return statement sends program flow out of the loop and out of the findName2() function.

- If no match is found in the StructureArray, then the for loop ends without ever resetting the value of structNum, and the function returns structNum = 0.

4	The findName2() function can now be used in the Command Window to find the row that contains the desired data. Here, we search for the name 'Steel'.

>> s = findName2('Steel',MetalProps);

s =

2

5	All that remains is to report the property values for steel.

>> Name = MetalProps(s).Name

Name =

Steel

>> Dens = MetalProps(s).Dens      % g/cm3

Dens =

7.8000

>> ModE = MetalProps(s).ModE      % GPa

ModE =

206.8000

>> ModR = MetalProps(s).ModR      % GPa

ModR =

80.8000

In script form, this example might look like this:

% Selecting Data from Structures in Matlab – Method 2

% File: Example7B.m

% 1 - First, clear any MetalProps that might be previously defined

clear MetalProps;

% 2 - Next, fill the structures in the structure array with data

%     (Property values from Table C-1)

MetalProps.Name = 'Alum';

MetalProps.Dens = 2.8;      % g/cm3

MetalProps.ModE = 71.7;     % GPa

MetalProps.ModR = 26.8;     % GPa

MetalProps(2).Name = 'Steel';

MetalProps(2).Dens = 7.8;      % g/cm3

MetalProps(2).ModE = 206.8;    % GPa

MetalProps(2).ModR = 80.8;     % GPa

MetalProps(3).Name = 'Copper';

MetalProps(3).Dens = 8.9;      % g/cm3

MetalProps(3).ModE = 120.7;    % GPa

MetalProps(3).ModR = 44.7;     % GPa

% 3 - create the findName2() function

% 4 - Find the data for 'Steel' in the MetalProps structure array

s = findName2('Steel',MetalProps);

% 5 - Report the results

Name = MetalProps(s).Name

Dens = MetalProps(s).Dens      % g/cm3

ModE = MetalProps(s).ModE      % GPa

ModR = MetalProps(s).ModR      % GPa

### Formatting Output in Matlab

Matlab provides one simple, but powerful function for formatting output: fprintf(). The fprintf() function is designed to control formatting when printing to a file. But, by setting the fileID (described later) to 2, the output goes to the computer screen rather than a file. In this way, the fprinft() function can be used to control the way values are presented on the screen.

The fprintf function needs three pieces of information:

- where to print (the fileID)

- how to print (the format string)

- what to print (the variable or value to be output)

The fileID is the return value of the fopen() function, which is used to open a disk file for printing. That's not going to be discussed here, because we will simply send output to the computer screen. Matlab has made this easy by saying that fileID = 2 is always the computer screen.

The format string provides information about how to display the printed value. A typical format string might look like this:

%10.3f

The percent symbol % is used to tell Matlab that format information follows. The 10.3 tells Matlab how many characters to provide for the output (10) and how many of them should be after the decimal point (3). The f stands for fixed-point notation (fixed number of decimal places). If the value of π was output using a %10.3f format, it would look like this:

□□□□□3.142

where the boxes represent blank spaces. The fprintf() function call to print π with a %10.3f format would look like this:

>> fileID = 2;

>> fprintf(fileID, '%10.3f \n', pi)

Explicitly declaring fileID = 2 is not required; you could simply put a 2 in the fprintf() function call. The \n in the format string tells Matlab to issue a line feed after printing the value. This causes the next values sent to the screen to appear below the value of π. Notice that the format string is enclosed in single quotes, as '%10.3f \n'. Single quotes are used in Matlab to identify text strings, and the format information is sent into the fprintf() function as a text string.

Here is how Matlab outputs the value of π without using fprintf().

>> pi

ans =

3.1416

Here is how Matlab outputs the value of π with fprintf() and a %10.3f format.

>> fprintf(2,'%10.3f \n',pi)

3.142

You can use the fprintf() function to include additional text by adding it to the format string. For example:

>> fprintf(2,'The value of pi is: %10.3f \n',pi)

The value of pi is:      3.142

For additional information on using the fprintf() function to control the appearance of output values the reader is directed to the Matlab help files. Looking up fprintf in the index will provide the additional information.

## Solving Simultaneous Linear Equations

Since Matlab is designed around matrix math, it seems appropriate to include an example of how to solve a system of simultaneous linear equations using matrix methods. This is especially important since the force and momentum balances that are an integral part of machine design commonly result in a set of linear equations that must be solved together to obtain results. For this example, we start with a simple set of simultaneous linear equations.

1 x1 + 3 x2 + 5 x3 + 7 x4 = 1

2 x1 + 1 x2 + 3 x3 + 5 x4 = 3

1 x1 + 5 x2 + 9 x3 + 2 x4 = 5

3 x1 + 1 x2 + 4 x3 + 1 x4 = 7

The first step is to rewrite this set of equations in matrix form, as a coefficient matrix times an x vector, set equal to a right-hand-side vector.

[C] [x] = [rhs]

The common method of solution is to invert the coefficient matrix and obtain the solution by multiplying [C]-1 and [rhs].

[C] [x] = [rhs]

[C]-1 [C] [x] = [C]-1 [rhs]

but [C]-1 [C] = [I], the identity matrix.

[I] [x] = [C]-1 [rhs]

Since multiplication by the identity matrix does not change the result,

[x] = [C]-1 [rhs]

This technique can be used in Matlab, however Matlab also offers an alternative technique that involves significantly fewer calculation steps. This translates to increased speed and less round-off error creeping into the solution. The technique is called Gaussian elimination with partial pivoting and is implemented in Matlab as left division. By way of definition, the solution to [C] [x] = [rhs] is [x] = [C]\[rhs].  Notice the backslash between [C] and [rhs]. The backslash is the operator that initiates left division.

The set of four simultaneous linear equations is solved using left division in the following example.

Example H-8

Solving Simultaneous Linear Equations in Matlab

Problem	Solve the set of four simultaneous linear equations using left division.

Given

1 x1 + 3 x2 + 5 x3 + 7 x4 = 1

2 x1 + 1 x2 + 3 x3 + 5 x4 = 3

1 x1 + 5 x2 + 9 x3 + 2 x4 = 5

3 x1 + 1 x2 + 4 x3 + 1 x4 = 7

Solution	See Matlab M-File Example 8.m.

1	First, establish the coefficient matrix.

>> Coeff = [1 3 5 7;

2 1 3 5;

1 5 9 2;

3 1 4 1]

Coeff =

1     3     5     7

2     1     3     5

1     5     9     2

3     1     4     1

The definition of Coeff was done on four rows for readability only; the coefficient matrix could be defined on a single line since the semicolons tell Matlab where the next row begins.

2	Establish the right-hand-side vector.

>> rhs = [1;3;5;7]

rhs =

1

3

5

7

3	Use "left division" to solve for the x vector.

>> x = Coeff\rhs

x =

1.6000

-0.6000

0.8000

-0.4000

The solution is complete at this point. However, as a check, we can multiply the coefficient matrix and the x vector to see if the values of the right-hand-side vector are produced.

4	Multiply the coefficient matrix and the x vector to see if the values of the right-hand-side vector are produced

>> r = Coeff * x

r =

1.0000

3.0000

5.0000

7.0000

The solution checks!

In script form, this example looks like this:

% % Simultaneous Linear Equations

% File: Example8.m

%

% 1 - Establish the coefficient matrix

Coeff = [1 3 5 7;

2 1 3 5;

1 5 9 2;

3 1 4 1];

% 2 - Establish the right-hand-side vector

rhs = [1;3;5;7]

% 3 - Use "left division" to solve for the x vector

x = Coeff\rhs

% 4 - Multiply coefficient matrix and x vector to check result

r = Coeff * x

## Introducing M-Books

M-Books are Microsoft Word documents with embedded, active Matlab commands. M-Books allow you to describe a solution and calculate the results in the same document. Moreover, the documents are interactive so that the reader can change input values to see what happens to the solution. This allows the documents to be used to "what if" scenarios. These features made M-Books the choice for presenting the Matlab solutions to the Examples and Case Studies in this book.

The process used to create the Example M-Books for this text was as follows (although other approaches could be used):

- A Matlab script m-file was prepared that contained the mathematical solution for the Example.

- A blank MS Word document was created by Matlab by issuing the  >> notebook  command in the Command Window. (Allowing Matlab to create the Word document establishes the connection between Word and Matlab, and adds the Notebook menu to MS Word. The Notebook menu provides the tools needed to turn text into Matlab statements.)

- The MS Word document was edited, creating the text to accompany the solution.

- The Matlab statements were copied and pasted from the script m-file into the Word document in appropriate locations.

- Each Matlab statement or group of Matlab statements was selected, and activated using the menu command Notebook / Define Input Cell (or the keyboard shortcut, [Alt-D]).

- The Matlab statements were evaluated together by using the menu command Notebook / Evaluate M-Book. At this step, all of the Matlab output was automatically inserted in the Word document.

- The M-Book was formatted for appearance, and to make the output sections stand out from the rest of the text.

If you have MS Word installed on your computer, you should be able to read the M-Books. However, in order to use M-Books interactively, you will need to have both MS Word and Matlab installed on your computer. When you open an M-Book in Word, Matlab will automatically start so that you can work with the Matlab equations in the document.